** Cell name: logic_125_BUFN

** View name: schematic

*. subckt logic_125_BUFN a gnd oe_n vcc y

*

**   model file

include .lib TT

.Param idc=2na vload=10v, vb1=0v vb2=5v va1=1V va2=2v cl=50pf vi=5v

*.Param v0=0v v11=0v v12=0v v1=0v v4=04

** Library name: SHRINK_74LVC1G_S026_sch

** Cell name: logic_pad_in

** View name: schematic

.subckt logic_pad_in out pad vss

*patch xr1 pad out rp1 r=200.07 l=83.9e-6 w=4e-6

r199 pad out 200

m1 out vss vss vss nl L=600e-9 W=20e-6 AD=23e-12 AS=23e-12 PD=42.3e-6 PS=42.3e-6 M=1

m0 pad vss vss vss nl L=600e-9 W=400.8e-6 AD=460.9e-12 AS=460.9e-12 PD=803.9e-6 PS=803.9e-6 M=1

.ends logic_pad_in

** End of subcircuit definition.

** Library name: SHRINK_74LVC1G_S026_sch

** Cell name: logic_pad_VCC

** View name: schematic

.subckt logic_pad_VCC out pad vss

*patch xr0 net13 vss rnw r=9.982e3 l=28.7e-6 w=4e-6

R499 net13 vss 10000

*patch xr2 net11 net13 rnw r=9.982e3 l=28.7e-6 w=4e-6

R500 net11 net1 10000

*patch xr1 pad out rp1 r=199.75 l=125.8e-6 w=6e-6

R198 pad out 200

m1 out vss vss vss nl L=600e-9 W=20e-6 AD=23e-12 AS=23e-12 PD=42.3e-6 PS=42.3e-6 M=1

m0 pad net11 vss vss nl L=600e-9 W=504e-6 AD=579.6e-12 AS=579.6e-12 PD=1.01e-3 PS=1.01e-3 M=1

.ends logic_pad_VCC

** End of subcircuit definition.

** Library name: SHRINK_74LVC1G_S026_sch

** Cell name: logic_OUT_PMOS

** View name: schematic

.subckt logic_OUT_PMOS b d g s

m46 d g s b pl L=600e-9 W=72e-6 AD=82.8e-12 AS=82.8e-12 PD=146.3e-6 PS=146.3e-6 M=1

.ends logic_OUT_PMOS

** End of subcircuit definition.

** Library name: SHRINK_74LVC1G_S026_sch

** Cell name: logic_OUT_NMOS

** View name: schematic

.subckt logic_OUT_NMOS b d g s

m47 d g s b nl L=600e-9 W=80e-6 AD=92e-12 AS=92e-12 PD=162.3e-6 PS=162.3e-6 M=1

.ends logic_OUT_NMOS

** End of subcircuit definition.

** Library name: SHRINK_74LVC1G_S026_sch

** Cell name: logic_125_BUFN

** View name: schematic

.subckt logic_125_BUFN a gnd oe_n vcc y

xi27 b_in oe_n gnd logic_pad_in

xi30 a_in a gnd logic_pad_in

xi26 vcc1 vcc gnd logic_pad_VCC

xi29_0 vb y pgate vcc logic_OUT_PMOS

xi29_1 vb y pgate vcc logic_OUT_PMOS

xi29_2 vb y pgate vcc logic_OUT_PMOS

xi29_3 vb y pgate vcc logic_OUT_PMOS

xi29_4 vb y pgate vcc logic_OUT_PMOS

xi29_5 vb y pgate vcc logic_OUT_PMOS

xi29_6 vb y pgate vcc logic_OUT_PMOS

xi29_7 vb y pgate vcc logic_OUT_PMOS

xi28_0 gnd y ngate gnd logic_OUT_NMOS

xi28_1 gnd y ngate gnd logic_OUT_NMOS

xi28_2 gnd y ngate gnd logic_OUT_NMOS

xi28_3 gnd y ngate gnd logic_OUT_NMOS

*patch xr1 net0200 y rp1 r=99.915 l=41.9e-6 w=4e-6

R197 net0200 y 100

*patch xr0 net0269 net0200 rp1 r=99.915 l=41.9e-6 w=4e-6

R196 net0269 net020 100

m3 b_in2 b_in gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=5

m36 a_in2 a_in gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=5

m26 a_in3 a_in2 gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=2

m29 ngate a_in3 gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=2

m40b ngate b_in3 gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=4

m38 net209 gnd gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=2

m27 net213 gnd gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=3

m48 net0425 gnd gnd gnd nl L=600e-9 W=90e-6 AD=103.5e-12 AS=103.5e-12 PD=182.3e-6 PS=182.3e-6 M=1

m39 net221 gnd gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=2

m32 a_in2 a_in gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=2

m4 b_in3 b_in2 gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=7

m28 ngate a_in3 gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=7

m12 pgate b_in2 net253 gnd nl L=600e-9 W=66e-6 AD=75.9e-12 AS=75.9e-12 PD=134.3e-6 PS=134.3e-6 M=1

m25 net0248 gnd gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=7

m49 vs b_in2 gnd gnd nl L=600e-9 W=24e-6 AD=27.6e-12 AS=27.6e-12 PD=50.3e-6 PS=50.3e-6 M=1

m31 a_in3 a_in2 gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=7

m43 net253 a_in3 gnd gnd nl L=600e-9 W=66e-6 AD=75.9e-12 AS=75.9e-12 PD=134.3e-6 PS=134.3e-6 M=1

m16 b_in2 b_in gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=2

m40a ngate b_in3 gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=5

m17 net265 gnd gnd gnd nl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=5

m1 net0269 gnd gnd gnd nl L=800e-9 W=20e-6 AD=23e-12 AS=23e-12 PD=42.3e-6 PS=42.3e-6 M=1

m18 net324 gnd vcc vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=11

m67b net217 gnd net284 vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=4

m67a net0489 gnd net284 vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=4

m33 a_in3 a_in2 vcc vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=11

m5 b_in3 b_in2 vcc vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=11

m35 a_in2 a_in vcc vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=3

m20 ngate b_in3 net300 vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=8

m57 pgate vcc1 y vb pl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=1

m68 vb net0269 vcc1 vb pl L=600e-9 W=24e-6 AD=27.6e-12 AS=27.6e-12 PD=50.3e-6 PS=50.3e-6 M=1

m2 vcc_int vs vcc vcc_int pl L=600e-9 W=220e-6 AD=253e-12 AS=253e-12 PD=442.3e-6 PS=442.3e-6 M=1

m22 pgate b_in2 vcc_int vcc_int pl L=600e-9 W=219e-6 AD=251.8e-12 AS=251.8e-12 PD=440.3e-6 PS=440.3e-6 M=1

m42 pgate a_in3 vcc_int vcc_int pl L=600e-9 W=219e-6 AD=251.8e-12 AS=251.8e-12 PD=440.3e-6 PS=440.3e-6 M=1

m21 net321 gnd net324 vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=11

m37 a_in2 a_in vcc vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=8

m24b net0333 gnd vcc vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=4

m58 vs vcc1 y vb pl L=600e-9 W=10e-6 AD=11.5e-12 AS=11.5e-12 PD=22.3e-6 PS=22.3e-6 M=1

m15 b_in2 b_in vcc vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=3

m0 b_in2 b_in vcc vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=8

m59 vb vs vcc vb pl L=600e-9 W=24e-6 AD=27.6e-12 AS=27.6e-12 PD=50.3e-6 PS=50.3e-6 M=1

m56 vb vcc1 y vb pl L=600e-9 W=20e-6 AD=23e-12 AS=23e-12 PD=42.3e-6 PS=42.3e-6 M=1

m66 net284 gnd vcc vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=8

m14 net368 gnd vcc vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=8

m19 net300 a_in3 vcc vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=8

m24a net365 gnd net368 vcc pl L=600e-9 W=12e-6 AD=13.8e-12 AS=13.8e-12 PD=26.3e-6 PS=26.3e-6 M=4

.ends logic_125_BUFN

** End of subcircuit definition.

**_** Library name: SHRINK_74LVC1G_S026_sch

**_** Cell name: 74LVC1G125_test

** View name: schematic

**_xi133 net021 vss net53 vcc1 n1y logic_125_BUFN

**_i1 y1 0 DC=idc

**_*v4 b 0 PULSE vb1 vb2 2.5e-9 2.5e-9 200e-9 400e-9

**_v4 b 0 PULSE 0 5 2.5e-9 2.5e-9 200e-9 400e-9

**_*v1 a 0 PULSE va1 va2 17e-9 2.5e-9 2.5e-9 22e-9 44e-9

**_v1 a 0 PULSE 0 5 17e-9 2.5e-9 2.5e-9 22e-9 44e-9

**_*c1 y1 0 cl

**_c1 y1 0 cl 50p

**_v12 net048 0 PWL (0 .1 1e-6 10)TD=0

**_*v12 net048 0 PWL (0 10 1e-6 10 1) td=0

**_*VCLOCK 7 5 PWL(0 ??7 10NS ??7 11NS ??3 17NS ??3 18NS ??7 50NS ??7) r **_=0 t d =15NS

**_*v11 net31 0 PWL 0 5 3e-6 vi *TD=0

**_v11 net31 0 PWL 0 5 3e-6 5 0

**_*v0 vcc 0 PWL 0 0 1.5e-6 vi *TD=0

**_v0 vcc 0 PWL 0 5 1.5e-6 vi 0

**_l54 b net41 2.7e-9

**_l55 a net39 2.7e-9

**_l48 vcc net37 2.7e-9

**_l42 n1y net35 2.7e-9

**_l49 0 net43 2.7e-9

**_r60 net39 net53 50e-3

**_r59 net41 net021 50e-3

**_r68 net31 net48 20

**_r73 net048 y1 500

**_r72 y1 0 500

**_r52 net35 y1 50e-3

**_r69 net48 0 20

**_r4 net43 vss 50e-3

**_r3 net37 vcc1 50e-3

******************************************************************************

******************************************************************************

**************************************************************************************

*                                                                                    *

*       IN THIS MODEL LIB CONTAINS :                                                 *

*                                                                                    *

*                                                                                    *

*       1.LIB TT                                                                     *

*

*        (Low Vt 5V N/PMOS,withdifferent corner models )                                                 *

*                                                                                    *

*                                                                                    *

*

*                                                                                    *

**************************************************************************************

******************************************************************************

******************************************************************************

**************************************************************************************

*       MODEL NAME Description :                                                     *

*                                                                                    *

*                                                                                    *

*       (1)MOS model :                                                               *

*                       model name      type                                         *

*                       -------------------------------------                        *

*                       nl              Low Vt NMOS                                  *

*                       pl              Low Vt PMOS                                  *

*                                                              *

*                         MOS LIB                              *

*                                                              *

****************************************************************

***************** CORNER_LIB OF TYPICAL MODEL ****************************

**.LIB TT

.PARAM

.param

+toxnh= 1.35E-08            toxph= 1.38E-08

+dxlnh= 0		          dxlph= 0

+dxwnh= 0    	          dxwph= 0

+dvtnh= 0      	          dvtph= 0

+dvsatnh= 1                 dvsatph= 1

+cjnh= 0.0005974            cjph= 0.0008878167

+cjswnh= 2.884E-10	    cjswph= 2.987026e-010

+cjswgnh= 1.7825E-10        cjswgph= 1.100685e-010

+cgonh= 2.00E-10            cgoph= 1.72e-010

+toxnl= 1.35E-08            toxpl= 1.38E-08

+dxlnl= 0                   dxlpl= 0

+dxwnl= 0                   dxwpl= 0

+dvtnl= 0                   dvtpl= 0

+dvsatnl= 1                 dvsatpl= 1

+cjnl= 5.82827E-04          cjpl= 0.000889

+cjswnl= 2.73309E-10        cjswpl= 3.03E-10

+cjswgnl= 1.84996E-10       cjswgpl= 3.13E-10

+cgonl= 2.00E-10	          cgopl= 1.72e-010

+dtoxNLD= 0

+dxlNLD= 0

+dxwNLD= 0

+dvtNLD= 0

+dvsatNLD= 1

+cjNLD= 5.95E-04

+cjswNLD= 2.96E-10

+cjswgNLD= 1.75E-10

+cgoNLD= 1.82E-10

+dtoxn5diso= 0

+dxln5diso= 0

+dxwn5diso= 0

+dvtn5diso= 0

+dvsatn5diso= 1

+cjn5diso= 5.95E-04

+cjswn5diso= 2.96E-10

+cjswgn5diso= 1.75E-10

+cgon5diso= 1.82E-10

+dtoxn5naiso= 0

+dxln5naiso= 0

+dxwn5naiso= 0

+dvtn5naiso= 0

+dvsatn5naiso= 1

+cjn5naiso= 5.79E-04

+cjswn5naiso= 3.19E-10

+cjswgn5naiso= 5.93E-10

+cgon5naiso= 1.9e-010

+dtoxp5naiso= 0

+dxlp5naiso= 0

+dxwp5naiso= 0

+dvtp5naiso= 0

+dvsatp5naiso= 1

+cjp5naiso= 9.33E-04

+cjswp5naiso= 3.53E-10

+cjswgp5naiso= 3.64E-10

+cgop5naiso= 1.8e-010

+dtoxn5na = 0             dxln5na = 0              dxwn5na = 0

+dvtn5na = 0                    dvsatn5na = 1

+cjn5na= 5.79E-04

+cjswn5na= 3.19E-10

+cjswgn5na= 5.93E-10

+cgon5na= 1.9e-010

+dtoxp5na = 0               dxlp5na = 0                dxwp5na = 0

+dvtp5na = 0                dvsatp5na = 1              cjp5na = 9.33E-4

+cjswp5na = 3.53E-10        cjswgp5na = 3.64E-10       cgop5na = 1.8E-10

+dtoxn5iso = 0               dxln5iso = 0                dxwn5iso = 0

+dvtn5iso = 0                dvsatn5iso = 1              cjn5iso = 5.79E-04

+cjswn5iso = 3.19E-10        cjswgn5iso = 5.93E-10       cgon5iso = 1.9e-010

+dtoxp5iso = 0               dxlp5iso = 0                dxwp5iso = 0

+dvtp5iso = 0                dvsatp5iso = 1              cjp5iso = 9.33E-4

+cjswp5iso = 3.53E-10        cjswgp5iso = 3.64E-10       cgop5iso = 1.8E-10

.lib 'c05logic5v.l' MOS

**.ENDL TT

*************************************************************************

***************************************************************

*                                                             *

*                 5V Low Vt NMOS DEVICE MODEL                 *

*                                                             *

***************************************************************

.model nl nmos

***** Flag Parameter ***

+level = 49

+binflag = 0                   Mobmod = 1                    Capmod = 3

+NQSMOD = 0

***** Geometry Range Parameter ***

+lmin = 6E-7                   lmax = 2E-4                   wmin = 5E-7

+wmax = 2E-4

***** Process Parameter ***

+Tox = 'toxnl'                 Toxm = 'toxnl'                Xj = 1.6E-7

+Nch = 1.1932E17

***** dW and dL Parameter ***

+wint = 9.855E-8               wl = -2.006178E-14            wln = 1

+ww = -7.327043E-26            wwn = 2.831724                wwl = 0

+lint = 6.251999E-8            ll = -8.686614E-12            lln = 0.598638

+lw = -2.5E-14                 lwn = 1                       lwl = 2.974177E-18

+Llc = 0                       Lwc = 0                       Lwlc = 0

+Wlc = 0                       Wwc = 0                       Wwlc = 0

+Dwg = -2.824E-9               Dwb = 1.122213E-8             Wmlt = 1

+Lmlt = 1                      xl = '-3.20E-09+dxlnl'        xw = '6.00E-09+dxwnl'

***** Vth Related  Parameter ***

+Vth0 = '0.449024+dvtnl'               lVth0 = 3.68E-8               pVth0 = 1.09746E-14

+Vfb = -1.020218               K1 = 0.739383                 lk1 = 1E-8

+pk1 = 6E-14                   K2 = 1.02145E-2               lk2 = 1.318E-9

+pK2 = -1.500881E-14           K3 = 0.1                      K3b = -0.32166

+W0 = 0                        Nlx = 3.959671E-7             Dvt0 = 6.540659

+Dvt1 = 0.562125               Dvt2 = -3.69695E-2            Dvt0w = 0.19999

+Dvt1w = 6.194009E5            Dvt2w = 0.18                  Ngate = 1E23

***** Mobility Related Parameter ***

+U0 = 4.75752E-2               lu0 = 8E-10                   pu0 = -9E-15

+Ua = -3.122E-10               lua = 3E-16                   pua = -2.4E-22

+Ub = 2.824674E-18             lub = -5.1E-25                pub = 1.72E-31

+Uc = 6.212605E-11             luc = 3.081488E-33            puc = 1.77503E-23

+Vsat = '1.02E5*dvsatnl'                 pVsat = 3.5219E-9             A0 = 1.054053

+la0 = -4.47737E-7             pa0 = -7.931266E-13           Ags = 0.228841

+lags = -1.449044E-6           pags = 5.578364E-12           B0 = 3.21E-6

+B1 = 1.52976E-5               Keta = -0.04093               lKeta = 1.358977E-8

+wKeta = 9.047501E-9           pKeta = -1.13836E-14          A1 = 0

+A2 = 0.99                     Rdsw = 562.535                pRdsw = 1.34098E-11

+Prwb = 0.100547               Prwg = 1.24443E-2             Wr = 0.85463

***** Subthreshold Related Parameter ***

+Voff = -0.102136              lvoff = 1.5E-8                pvoff = -2.4032E-14

+NFactor = 1.147517            Eta0 = 0.0968                 Etab = -0.0144

+Dsub = 0.431185               Cit = -1.402944E-4            Cdsc = 2.514096E-4

+Cdscb = -1.148242E-4          Cdscd = 0

***** Output Resistance Related Parameter ***

+Pclm = 0.4023                 Pdiblc1 = 1.388293E-3         Pdiblc2 = 8.56927E-4

+Pdiblcb = 7.47873E-2          Drout = 4.32117E-2            Pscbe1 = 3.72678E8

+Pscbe2 = 1E-20                Pvag = 0.111161               Delta = 1.03425E-2

+Alpha0 = 9.26E-7              Alpha1 = 7.472                Beta0 = 33.699998

***** NQS Parameter ***

+Elm = 5

***** Capacitance Parameter ***

+Xpart = 0                     Cgso = 'cgonl'                Cgdo = 'cgonl'

+Cgbo = 1E-13                  ckappa = 0.6                  cf = 0

+clc = 5E-8                    cle = 0.51                    Dlc = 1.23E-8

+Dwc = 0                       Vfbcv = -0.780837             Noff = 4

+Voffcv = -0.3283              Acde = 1                      Moin = 15

+cgsl = 2.85E-11               cgdl = 2.85E-11

***** Souce/Drain Junction Diode Model Parameter ***

+calcacm = 1                   ACM = 12                      ldif = 1.4E-7

+hdif = 5.1E-7                 rsh = 69.4                    rd = 0

+rs = 0                        rsc = 0                       rdc = 0

+Cj = 'cjnl'                   Mj = 0.392394                 Mjsw = 0.181217

+Cjsw = 'cjswnl'               Cjswg = 'cjswgnl'             Mjswg = 0.476504

+JS = 3.33E-4                  JSW = 6.315E-10               Pbsw = 0.460303

+Pb = 0.785694                 Pbswg = 0.84988

***** Temperature Coeffient ***

+Tref = 25                     Ute = -1.226591               lute = 1E-8

+wUte = 9E-8                   pUte = -8.875E-14             kt1 = -0.300654

+lkt1 = -2.2E-8                Kt1l = -4.40444E-8            kt2 = -0.137092

+lkt2 = 4.11276E-8             Ua1 = 4.9184E-9               lua1 = 6E-17

+pua1 = -3.6888E-21            Ub1 = -7.309219E-18           lub1 = -2.37E-25

+pub1 = 6.981914E-30           Uc1 = 4.216E-11               At = 2.142E4

+Prt = 77.31898                pprt = -7.230665E-10          Nj = 1

+Xti = 3                       Tpb = 1.996955E-3             Tpbsw = 1.648795E-3

+Tpbswg = 1.82E-3              Tcj = 1.278398E-3             Tcjsw = 8.744448E-4

+Tcjswg = 1.99E-3

***************************************************************

*                                                             *

*                 5V Low Vt PMOS DEVICE MODEL                 *

*                                                             *

***************************************************************

.model pl pmos

***** Flag Parameter ***

+level = 49

+binflag = 0                   mobmod = 1                    capmod = 3

+nqsmod = 0                    noimod = 1

***** Geometry Range Parameter ***

+lmin = 6E-7                   lmax = 2E-4                   wmin = 5E-7

+wmax = 2E-4

***** Process Parameter ***

+tox = 'toxpl'                 toxm = 'toxpl'                xj = 1.7E-7

+nch = 4.5405E16

***** dW and dL Parameter ***

+wint = 0                      wl = 0                        wln = 1

+ww = -3.1808E-14              wwn = 1                       wwl = 0

+lint = 1E-9                   ll = 3.5393E-11               lln = 0.29998

+lw = 0                        lwn = 1                       lwl = 0

+llc = 0                       lwc = 0                       lwlc = 0

+wlc = 0                       wwc = 0                       wwlc = 0

+dwg = -1E-8                   dwb = 6E-9                    wmlt = 1

+lmlt = 1                      xl = '1.4e-007+dxlpl'         xw = '0+dxwpl'

***** Vth Related  Parameter ***

+vth0 = '-0.6544+dvtpl'                lvth0 = 8.498992E-8               wvth0 = 2.7196E-8

+pvth0 = 1.754625E-14            vfb = -0.337584               k1 = 0.643248

+lk1 = 2.7E-8                  k2 = 0.065036                 lk2 = -3.1777E-8

+k3 = 25.827                   k3b = -3.9253                 w0 = 1.3101E-6

+nlx = 6.6711E-7               dvt0 = 2                      dvt1 = 0.42575

+dvt2 = -0.05                  dvt0w = 0.218247              dvt1w = 5.3E6

+dvt2w = -0.032                ngate = 1E23

***** Mobility Related Parameter ***

+u0 = 0.01722                  lu0 = 1.809458E-25            wu0 = -4.8639E-9

+pu0 = 3.36072E-16             ua = 1.223865E-9              lua = -4.945E-16

+pua = 5E-24                   ub = 7.767531E-19             lub = 6.064612E-25

+pub = -6.1967E-33             uc = -4.16E-11                puc = 4E-24

+vsat = '8.2256E4*dvsatpl'     a0 = 1.4292                   la0 = -1.064293E-6

+pa0 = -9.79553E-13             ags = 0.131                   lags = -4.6959E-8

+pags = 1.096875E-12           b0 = 8E-7                     b1 = 1E-6

+keta = -3E-3                  lketa = -2.2675E-8            wketa = 1.5165E-8

+pketa = -7.3375E-16           a1 = 0                        a2 = 1

+rdsw = 1.2042E3               prdsw = -4.5E-11              prwb = -0.08

+prwg = -2.75E-4               wr = 1

***** Subthreshold Related Parameter ***

+voff = -0.181584              lvoff = 6E-9                  nfactor = 2.05029

+lnfactor = -8.320565E-7       wnfactor = 7.15E-7            eta0 = 0.08

+etab = -0.07                  dsub = 0.99501                cit = 1.1E-5

+cdsc = 1.10298E-3             cdscb = 4.282499E-4           cdscd = 1.9E-4

***** Output Resistance Related Parameter ***

+pclm = 1.99773                pdiblc1 = 1.35                pdiblc2 = 3.15E-3

+pdiblcb = -1E-3               drout = 0.945                 pscbe1 = 3.575993E8

+pscbe2 = 1E-5                 pvag = 0.4                    delta = 0.01

+alpha0 = 5.3E-8               palpha0 = 1.02E-20            alpha1 = 0.2057

+lalpha1 = 1.330004E-7         beta0 = 29.99085              pbeta0 = 1.13625E-13

***** Flicker Noise Model Parameter ***

+af = 1                        ef = 1.07                     kf = 1.995E-30

***** NQS Parameter ***

+elm = 5

***** Capacitance Parameter ***

+xpart = 0                     cgso = 'cgopl'                cgdo = 'cgopl'

+cgbo = 1E-13                  ckappa = 0.6                  cf = 0

+clc = 4.67E-11                cle = 0.751                   dlc = 8E-9

+dwc = -1E-23                  vfbcv = -0.32                 noff = 4

+voffcv = -0.1                 acde = 1                      moin = 15

+cgsl = 1.03E-11               cgdl = 1.03E-11

***** Souce/Drain Junction Diode Model Parameter ***

+calcacm = 1                   acm = 12                      ldif = 1.4E-7

+hdif = 5.1E-7                 rsh = 143                     rd = 0

+rs = 0                        rsc = 0                       rdc = 0

+cj = 'cjpl'                   mj = 0.524                    mjsw = 0.297

+cjsw = 'cjswpl'               cjswg = 'cjswgpl'             mjswg = 0.103

+js = 7.9E-5                   jsw = 6.92E-11                pbsw = 0.737

+pb = 1.1                      pbswg = 0.263

***** Temperature Coeffient ***

+tref = 25                     ute = -1.1995                 lute = 2.3E-8

+wute = 0                      pute = 2E-13                  kt1 = -0.685563

+lkt1 = -8E-9                  kt1l = 3.1048E-8              kt2 = -0.21012

+lkt2 = 1.0506E-7              ua1 = 9.87825E-10             lua1 = -8.7E-17

+pua1 = -1E-23                 ub1 = -1.203905E-18           lub1 = -2.43415E-25

+pub1 = 0                      uc1 = -4.59996E-13            at = 1E4

+prt = 100                     pprt = -2.22E-10              nj = 1

+xti = 3                       tpb = 1.94E-3                 tpbsw = 1.8E-3

+tpbswg = 0                    tcj = 1.11E-3                 tcjsw = 8.96E-4

+tcjswg = 2.6E-5

.ENDL MOS