simulator lang=spectre insensitive=yes

//   **********************************************

//   *             nuvoTon SPICE MODEL            *

//   **********************************************

//

//  ******************************

//        UPDATE HISTORY         *

//  ******************************

//

//   2012.12.11 A.L.Tsou - Version_A.

//   The model is 0.35um 3.3V/5V SPTM/DPTM logic process MACRO SPICE model.

//   The model card includes (1)3.3V N/PMOS.(2)3.3V ISO N/PMOS.(3)5V N/PMOS.(4)5V ISO N/PMOS.

//                   (5)3.3V Low Vt N/PMOS.(6)5V Low Vt N/PMOS.((7)BJTX1.(8)DiodeX4.(9)ResistorX17.

//			  		(10)POP capacitorX1.(11)1~2P/1~3M Interconnect.

//   All models are copied from 5050-1123(0.35um 3.3V/5V SPTM/DPTM logic process) except 3.3V Low Vt PMOS.

//

//   2013.02.01 W.H.Tseng - Version_B.

//   1) Add pure 3.3V PNP BJT(qpl8)

//

//   2013.06.07 W.H.Tseng - Version_C.

//   1) Modify the width trend for 3.3V low Vt N/PMOS

//

//   2013.10.04 C.J.Chen - Version_D

//   1) remodel 1K, 2K, 4K HR poly2 resistor

//   2) remodel P+/NW Diode (5um*5um)

//   3) Revise LR Poly2 model (copy from 5050-1125)

//

//   2013.11.15 C.J.Chen - Version_E

//   1) add 10K HR poly2 resistor

//

//   2014.07.14 A.L.Tsou - Version_F

//   1) Maintain the interconnect model.

//   2) Revise the DM process interconnect model (M2:8K->6K).

//

//  ******************************************************************

//        0.35um 3.3V/5V SPTM/DPTM logic process (HSPICE level=49 )  *

//  ******************************************************************

//

//    Design rule: 5050-1130

//    PROCESS route: C035DI & C031RI

//     Lot No. :S61897700 #9 for 3.3V/5V NMOS and PMOS

//              28459730#7   for 3.3V low-Vt NMOS and low-Vt N+ Rs

//              28459730#23  for 3.3V low-Vt PMOS , low-Vt P+ Rs

//              2724990 #7   for N+/PW, P+/NW diode and Poly2 Rs

//              S6279280#8   for 3.3V low vt PMOS

//              S6359010#1   for P+/NW Diode (5um*5um)

//              S6279280#6   for pure 3.3V PNP BJT (qpl8)

//              S6339800#1   for 1K HR poly2

//              S6339800#2   for 2K HR poly2

//              S6339800#5   for 4K HR poly2

//              S6339800#11  for 10K HR poly2

//              22469170#3   for LR Poly2

//     Testkey Manunal :  BZ5948 & TK5163 & TK5346 & TK5543

//     Temperature : 	-40 C ~ 150 C

//     SPECTRE version:	10.1.1.260

//     Model Level : 	BSIM3V3.2 for MOS model

//                    	Gummel-Poon model for BJT

//

//  ****************************************************************************

//  ****************************************************************************

//  ************************************************************************************

//  *                                                                                  *

//  *      IN THIS MODEL LIB CONTAINS :                                                *

//  *      1.LIB TT                                                                    *

//  *            SS                                                                    *

//  *            FF                                                                    *

//  *            SF                                                                    *

//  *            FS                                                                    *

//  *       ( 3.3V Normal, isolation and low-Vt devices with different geometric and   *

//  *         corner models)                                                           *

//  *                                                                                  *

//  *      2.LIB TT_hv                                                                 *

//  *            SS_hv                                                                 *

//  *            FF_hv                                                                 *

//  *            SF_hv                                                                 *

//  *            FS_hv                                                                 *

//  *       ( 5V Normal, isolation and low Vt devices with different geometric and corner *

//  *         models)                                                                  *

//  *                                                                                  *

//  *      3.LIB T_bip                                                                 *

//  *            S_bip                                                                 *

//  *            F_bip                                                                 *

//  *       ( P+/NW/Psub 8x8 vertical PNP bipolar)                                     *

//  *                                                                                  *

//  *      4.LIB DIO                                                                   *

//  *       ( N+/PW, P+/NW , P+/NW(5um*5um),and P+/NW Zener diodes)                    *

//  *                                                                                  *

//  *      5.LIB T_res                                                                 *

//  *            S_res                                                                 *

//  *            F_res                                                                 *

//  *       ( N+, P+, low-Vt N+/P+, poly1, NW, HVNW, HR Poly2, LR Poly2, HR poly2, M1, *

//  *         M2 , rtm6k, rtm8k , rtm20k, rtm30k resistors)                            *

//  *                                                                                  *

//  *      6.LIB T_cap                                                                 *

//  *            S_cap                                                                 *

//  *            F_cap                                                                 *

//  *       (POP capacitor)                                                            *

//  *                                                                                  *

//  ************************************************************************************

//  ****************************************************************************

//  ****************************************************************************

//  ************************************************************************************

//  *      MODEL NAME Description :                                                    *

//  *                                                                                  *

//  *                                                                                  *

//  *      (1)MOS model :                                                              *

//  *                      model name      type                                        *

//  *                      -------------------------------------                       *

//  *                      n3              3.3v NMOS                                   *

//  *                      p3              3.3v PMOS                                   *

//  *                      n3iso           3.3V iso NMOS                               *

//  *                      p3iso           3.3V iso PMOS                               *

//  *                      n5              5.0v NMOS                                   *

//  *                      p5              5.0v PMOS                                   *

//  *                      n5iso           5.0V iso NMOS                               *

//  *                      p5iso           5.0V iso PMOS                               *

//  *                      nl              3.3V Low Vt NMOS                            *

//  *                      pl              3.3V Low Vt PMOS                            *

//  *                      n5l             5.0V low Vt NMOS                            *

//  *                      p5l             5.0V low Vt PMOS                            *

//  *                                                                                  *

//  *      (2)BJT model :                                                              *

//  *                      model name      type                                        *

//  *                      -------------------------------------                       *

//  *                      qp8             P+/NW/Psub(8x8)                             *

//  *                      qpl8            P+/NW/Psub(8x8)for pure 3.3V PNP BJT        *

//  *                                                                                  *

//  *      (3)Diode model :                                                            *

//  *                      model name      type                                        *

//  *                      -------------------------------------                       *

//  *                      dn              N+/PW Diode                                 *

//  *                      dp              P+/NW Diode                                 *

//  *                      dp2             P+/NW Diode (5um*5um)                       *

//  *                      dz              P+/NW Zener Diode                           *

//  *                                                                                  *

//  *      (4)Resistor model :                                                         *

//  *                          model name      type                                    *

//  *                          ---------------------------------                       *

//  *                          rnp             N+                                      *

//  *                          rpp             P+                                      *

//  *                          rnpl            low-Vt N+                               *

//  *                          rppl            low-Vt P+                               *

//  *                          rp1             Poly1                                   *

//  *                          rnw             NW                                      *

//  *                          rhnw            HVNW                                    *

//  *                          rhr1k           1K HR Poly2                             *

//  *                          rhr2k           2K HR Poly2                             *

//  *                          rhr4k           4K HR Poly2                             *

//  *                          rhr10k          10K HR Poly2                            *

//  *                          rlp2            LR Poly2 w/i 1K HR                      *

//  *                          rp2             Poly2                                   *

//  *                          rm1             M1                                      *

//  *                          rm2             M2                                      *

//  *                          rtm6k           6K Top Metal                            *

//  *                          rtm8k           8K Top Metal                            *

//  *                          rtm20k          20K Top Metal                           *

//  *                          rtm30k          30K Top Metal                           *

//  *                                                                                  *

//  *      (5)pop model :                                                              *

//  *                          model name      type                                    *

//  *                          ---------------------------------                       *

//  *                          cp             POP (W*L= 50um*120um)                    *

//  *                                                                                  *

//  *                                                                                  *

//  ************************************************************************************

//

//   ***********************************************

//   *                (1)MOS MODEL                 *

//   ***********************************************

//

//  Device type :

//     Low   V : 3.3V NMOS & PMOS ; 3.3V Low Vt N/PMOS

//     High  V : 5.0V NMOS & PMOS

//

//  Geometry range for each model :

//                                        width          length

//   type              model_name       low   high     low    high

//  ---------------------------------------------------------------

//   3.3V NMOS         n3               0.4u  200u     0.35u  200u

//   3.3V PMOS         p3               0.4u  200u     0.35u  200u

//   3.3V iso NMOS     n3iso            0.4u  200u     0.4u   200u

//   3.3V iso PMOS     p3iso            0.4u  200u     0.35u  200u

//   5V NMOS           n5               0.4u  200u     0.5u   200u

//   5V PMOS           p5               0.4u  200u     0.5u   200u

//   5V iso NMOS       n5iso            0.4u  200u     0.5u   200u

//   5V iso PMOS       p5iso            0.4u  200u     0.5u   200u

//   3.3V low-Vt NMOS  nl               0.4u  200u     0.35u  200u

//   3.3V low-Vt PMOS  pl               0.4u  200u     0.35u  200u

//   5V low-Vt NMOS    n5l              0.4u  200u     0.5u   200u

//   5V low-Vt PMOS    p5l              0.4u  200u     0.5u   200u

//

//  Fitting Voltage Condition :

//      |VG|=0~3.3V,|VD|=0~3.3V,|VB|=0~3.3V For 3.3V MOS

//      |VG|=0~5V,|VD|=0~5V,|VB|=0~3.3V For 5V MOS

//

//

//  corner_name :

//      TT(TT_hv) : typical model

//      SS(SS_hv) : slow NMOS, slow PMOS model

//      FF(FF_hv) : fast NMOS, fast PMOS model

//      SF(SF_hv) : slow NMOS, fast PMOS model

//      FS(FS_hv) : fast NMOS, slow PMOS model

//

//  Please include these library by adding a command into your input file.

//     Ex: to include TYPICAL model

//         # .lib '/your_dir_path/c035logic3v5v.scs

//

//  Simulated electrical parameters from corner models:

//  Here, Vth is extracted from HSPICE LV9() function. All values for PMOS

//  must add a negative sign. [Unit: Isat(mA),Vth(V)]

//

//                               TT             SS            FF            SF             FS

//    model_name    size      Idsat/Vth      Idsat/Vth     Idsat/Vth     Idsat/Vth      Idsat/Vth

// ---------------------------------------------------------------------------------------------------

//    n3(25C)       20/0.35   10.4m/0.65     8.40m/0.771   12.4m/0.53    8.40m/0.771	12.4m/0.53

//    p3(25C)       20/0.35   4.8m/0.84      3.3m/0.962    6.3m/0.72     6.3m/0.72      3.3m/0.962

//    n3iso(25C)    20/0.4    8.74m/0.68     6.99m/0.8     10.49m/0.56   6.99m/0.8	 10.49m/0.56

//    p3iso(25C)    20/0.35   4.48m/0.84     3.58m/0.96    5.38m/0.72    5.38m/0.72    3.58m/0.96

//    n5(25C)       20/0.5    9.64m/0.842    7.92m/1.003   11.3m/0.700   7.92m/1.003    11.3m/0.700

//    p5(25C)       20/0.5    4.56m/1.05     3.52m/1.229   5.53m/0.941   5.53m/0.941    3.52m/1.229

//    n5iso(25C)    20/0.55   8.72m/0.91     6.98m/1.06    10.46m/0.76   6.98m/1.06	 10.46m/0.76

//    p5iso(25C)    20/0.5    4.74m/1.12     3.79m/1.27    5.69m/0.97    5.69m/0.97    3.79m/1.27

//    nl(25C)       20/0.35   9.4m/0.6       7.52m/0.72    11.28m/0.48   7.52m/0.72    11.28m/0.48

//    pl(25C)       20/0.35   3.56m/0.67     2.85m/0.79    4.27m/0.55    4.27m/0.55    2.85m/0.79

//    n5l(25C)      20/0.5    8.7m/0.8       6.7m/0.95     10.7m/0.65    6.7m/0.95     10.7m/0.65

//    p5l(25C)      20/0.5    5m/0.9         4m/10.5       6m/0.75       6m/0.75       4m/10.5

//  **************************************************************************************************

//

//

//   ***********************************************

//   *                (2)BJT MODEL                 *

//   ***********************************************

//

//  Operation voltage: |Vc|=0V~5V, |Vb|=0.2V~1V, |Ve|=0V

//

//   ***********************************************

//   *                (3)Diode MODEL               *

//   ***********************************************

//

//  Operation Voltage: |VF|=0V~1V,|VR|=0V~10V for P+/NW Zener Diode

//                     |VF|=0V~1V,|VR|=0V~12V for N+/PW, P+/NW , P+/NW(5um*5um) Diode

//

//   ***********************************************

//   *                (4)Resistor MODEL            *

//   ***********************************************

//

//   fitting range:

//                 rnp: W/L from 0.4um/100um to 10um/100um

//                 rpp: W/L from 0.4um/100um to 10um/100um

//                rnpl: W/L only 15um/120um

//                rppl: W/L only 15um/120um

//                 rp1: W/L from 0.35/100um to 10um/100um

//                 rnw: W/L from 4um/100um to 10um/100um

//                rhnw: W/L from 4um/100um to 10um/100um

//                rhr1k: W/L from 5.0um/100um to 10um/100um

//                rhr2k: W/L from 5.0um/100um to 10um/100um

//                rhr4k: W/L from 5.0um/100um to 10um/100um

//                rp2 : W/L only 15um/120um

//                rlp2: W/L from 1.0um/100um to 10um/100um

//

//

//  Note: 1.Either (rp2) model or (rlp2,rhr1k,rhr2k,rhr4k) model can be used when designing. Both of them can not exist at the same time.

//        2.Please contact CE of the NUVOTON foundry before using rp2 model.

//        3.Low-Vt Nplus Resistor(rnpl) and Low-Vt Pplus Resistor(rppl) are only for 3.3V low-Vt device

//        4.PNP BJT(qpl8) is used for 3.3V low vt process

//

//  ****************************************************************************

//  ****************************************************************************

//

//  *************************************************************

//  *                                                           *

//  *     3.3V NORMAL, isolation and LOW-VT MOS DEVICES LIB     *

//  *                                                           *

//  *************************************************************

//

//  *************** CORNER_LIB OF TYPICAL MODEL ****************************

//

library MyLib

section tt

parameters

+toxn3v = 8.566e-9             toxp3v = 8.652e-9             dxln3v = 0

+dxlp3v = 0                    dxwn3v = 0                    dxwp3v = 0

+dvtn3v = 0                    dvtp3v = 0                    dvsatn3v = 1

+du0p3v = 1                    cjn3v = 8.278441e-4           cjp3v = 1.446915e-3

+cjswn3v = 2.071174e-10        cjswp3v = 3.226717e-11        cjswgn3v = 1.414663e-10

+cjswgp3v = 1.614054e-10       cgon3v = 1.6e-10              cgop3v = 2.9e-10

+dtoxn3iso = 0                 dtoxp3iso = 0                 dxln3iso = 0

+dxlp3iso = 0                  dxwn3iso = 0                  dxwp3iso = 0

+dvtn3iso = 0                  dvtp3iso = 0                  du0n3iso = 1

+dvsatp3iso = 1                cjn3iso = 8.278441e-4         cjp3iso = 1.446915e-3

+cjswn3iso = 2.071174e-10      cjswp3iso = 3.226717e-11      cjswgn3iso = 1.414663e-10

+cjswgp3iso = 1.614054e-10     cgon3iso = 1.6e-10            cgop3iso = 1.94e-10

+dtoxn3 = 0                    dtoxp3 = 0                    dxln3 = 0

+dxlp3 = 0                     dxwn3 = 0                     dxwp3 = 0

+dvtn3 = 0                     dvtp3 = 0                     du0n3 = 1

+dvsatp3 = 1                   cjn3 = 8.278441e-4            cjp3 = 1.446915e-3

+cjswn3 = 2.071174e-10         cjswp3 = 3.226717e-11         cjswgn3 = 1.414663e-10

+cjswgp3 = 1.614054e-10        cgon3 = 1.6e-10               cgop3 = 1.94e-10

include "c035logic3v5v.scs" section=mos

endsection tt

//  ************** CORNER_LIB OF  SNSP   MODEL *****************************

section ss

parameters

+toxn3v = 9.266e-9             toxp3v = 9.352e-9             dxln3v = 4e-8

+dxlp3v = 4e-8                 dxwn3v = -1.1e-8              dxwp3v = -1.1e-8

+dvtn3v = 0.0925               dvtp3v = -0.1225              dvsatn3v = 0.9085

+du0p3v = 0.8435               dtoxn3iso = 1e-9              dtoxp3iso = 1e-9

+dxln3iso = 4e-8               dxlp3iso = 4e-8               dxwn3iso = -6e-8

+dxwp3iso = -6e-8              dvtn3iso = 0.137              dvtp3iso = -0.13265

+du0n3iso = 0.843              dvsatp3iso = 0.943            dtoxn3 = 1e-9

+dtoxp3 = 1e-9                 dxln3 = 5e-9                  dxlp3 = 5e-9

+dxwn3 = -6e-8                 dxwp3 = -6e-8                 dvtn3 = 0.129

+dvtp3 = -0.1295               du0n3 = 0.79115               dvsatp3 = 0.786

parameters

+cjn3v = 8.278441e-04*1.1      cjp3v = 1.446915e-03*1.1      cjswn3v = 2.071174e-10*1.1

+cjswp3v = 3.226717e-11*1.1    cjswgn3v = 1.414663e-10*1.1   cjswgp3v = 1.614054e-10*1.1

+cgon3v = 1.6e-10*1.1          cgop3v = 2.9e-10*1.1          cjn3iso = 8.278441e-04*1.1

+cjp3iso = 1.446915e-03*1.1    cjswn3iso = 2.071174e-10*1.1  cjswp3iso = 3.226717e-11*1.1

+cjswgn3iso = 1.414663e-10*1.1 cjswgp3iso = 1.614054e-10*1.1 cgon3iso = 1.6e-10*1.1

+cgop3iso = 1.94e-10*1.1       cjn3 = 8.278441e-04*1.1       cjp3 = 1.446915e-03*1.1

+cjswn3 = 2.071174e-10*1.1     cjswp3 = 3.226717e-11*1.1     cjswgn3 = 1.414663e-10*1.1

+cjswgp3 = 1.614054e-10*1.1    cgon3 = 1.6e-10*1.1           cgop3 = 1.94e-10*1.1

include "c035logic3v5v.scs" section=mos

endsection ss

//  ************** CORNER_LIB OF  FNFP   MODEL ****************************

//

section ff

parameters

+toxn3v = 7.866e-9             toxp3v = 7.952e-9             dxln3v = -4e-8

+dxlp3v = -4e-8                dxwn3v = 1.1e-8               dxwp3v = 1.1e-8

+dvtn3v = -0.0865              dvtp3v = 0.123                dvsatn3v = 1.04

+du0p3v = 1.075                dtoxn3iso = -1e-9             dtoxp3iso = -1e-9

+dxln3iso = -4e-8              dxlp3iso = -4e-8              dxwn3iso = 6e-8

+dxwp3iso = 6e-8               dvtn3iso = -0.136             dvtp3iso = 0.1341

+du0n3iso = 0.999              dvsatp3iso = 0.9391           dtoxn3 = -1e-9

+dtoxp3 = -1e-9                dxln3 = -5e-9                 dxlp3 = -5e-9

+dxwn3 = 6e-8                  dxwp3 = 6e-8                  dvtn3 = -0.12878

+dvtp3 = 0.1293                du0n3 = 1.1424                dvsatp3 = 1.1461

parameters

+cjn3v = 8.278441e-04*0.9      cjp3v = 1.446915e-03*0.9      cjswn3v = 2.071174e-10*0.9

+cjswp3v = 3.226717e-11*0.9    cjswgn3v = 1.414663e-10*0.9   cjswgp3v = 1.614054e-10*0.9

+cgon3v = 1.6e-10*0.9          cgop3v = 2.9e-10*0.9          cjn3iso = 8.278441e-04*0.9

+cjp3iso = 1.446915e-03*0.9    cjswn3iso = 2.071174e-10*0.9  cjswp3iso = 3.226717e-11*0.9

+cjswgn3iso = 1.414663e-10*0.9 cjswgp3iso = 1.614054e-10*0.9 cgon3iso = 1.6e-10*0.9

+cgop3iso = 1.94e-10*0.9       cjn3 = 8.278441e-04*0.9       cjp3 = 1.446915e-03*0.9

+cjswn3 = 2.071174e-10*0.9     cjswp3 = 3.226717e-11*0.9     cjswgn3 = 1.414663e-10*0.9

+cjswgp3 = 1.614054e-10*0.9    cgon3 = 1.6e-10*0.9           cgop3 = 1.94e-10*0.9

include "c035logic3v5v.scs" section=mos

endsection ff

//  ************** CORNER_LIB OF  SNFP   MODEL ****************************

section sf

parameters

+toxn3v = 9.266e-9             toxp3v = 7.952e-9             dxln3v = 4e-8

+dxlp3v = -4e-8                dxwn3v = -1.1e-8              dxwp3v = 1.1e-8

+dvtn3v = 0.0925               dvtp3v = 0.123                dvsatn3v = 0.9085

+du0p3v = 1.075                dtoxn3iso = 1e-9              dtoxp3iso = -1e-9

+dxln3iso = 4e-8               dxlp3iso = -4e-8              dxwn3iso = -6e-8

+dxwp3iso = 6e-8               dvtn3iso = 0.137              dvtp3iso = 0.1341

+du0n3iso = 0.843              dvsatp3iso = 0.9391           dtoxn3 = 1e-9

+dtoxp3 = -1e-9                dxln3 = 5e-9                  dxlp3 = -5e-9

+dxwn3 = -6e-8                 dxwp3 = 6e-8                  dvtn3 = 0.129

+dvtp3 = 0.1293                du0n3 = 0.79115               dvsatp3 = 1.1461

parameters

+cjn3v = 8.278441e-04*1.1      cjp3v = 1.446915e-03*0.9      cjswn3v = 2.071174e-10*1.1

+cjswp3v = 3.226717e-11*0.9    cjswgn3v = 1.414663e-10*1.1   cjswgp3v = 1.614054e-10*0.9

+cgon3v = 1.6e-10*1.1          cgop3v = 2.9e-10*0.9          cjn3iso = 8.278441e-04*1.1

+cjp3iso = 1.446915e-03*0.9    cjswn3iso = 2.071174e-10*1.1  cjswp3iso = 3.226717e-11*0.9

+cjswgn3iso = 1.414663e-10*1.1 cjswgp3iso = 1.614054e-10*0.9 cgon3iso = 1.6e-10*1.1

+cgop3iso = 1.94e-10*0.9       cjn3 = 8.278441e-04*1.1       cjp3 = 1.446915e-03*0.9

+cjswn3 = 2.071174e-10*1.1     cjswp3 = 3.226717e-11*0.9     cjswgn3 = 1.414663e-10*1.1

+cjswgp3 = 1.614054e-10*0.9    cgon3 = 1.6e-10*1.1           cgop3 = 1.94e-10*0.9

include "c035logic3v5v.scs" section=mos

endsection sf

//  ************** CORNER_LIB OF  FNSP   MODEL ****************************

//

section fs

parameters

+toxn3v = 7.866e-9             toxp3v = 9.352e-9             dxln3v = -4e-8

+dxlp3v = 4e-8                 dxwn3v = 1.1e-8               dxwp3v = -1.1e-8

+dvtn3v = -0.0865              dvtp3v = -0.1225              dvsatn3v = 1.04

+du0p3v = 0.8435               dtoxn3iso = -1e-9             dtoxp3iso = 1e-9

+dxln3iso = -4e-8              dxlp3iso = 4e-8               dxwn3iso = 6e-8

+dxwp3iso = -6e-8              dvtn3iso = -0.136             dvtp3iso = -0.13265

+du0n3iso = 0.999              dvsatp3iso = 0.943            dtoxn3 = -1e-9

+dtoxp3 = 1e-9                 dxln3 = -5e-9                 dxlp3 = 5e-9

+dxwn3 = 6e-8                  dxwp3 = -6e-8                 dvtn3 = -0.12878

+dvtp3 = -0.1295               du0n3 = 1.1424                dvsatp3 = 0.786

parameters

+cjn3v = 8.278441e-04*0.9      cjp3v = 1.446915e-03*1.1      cjswn3v = 2.071174e-10*0.9

+cjswp3v = 3.226717e-11*1.1    cjswgn3v = 1.414663e-10*0.9   cjswgp3v = 1.614054e-10*1.1

+cgon3v = 1.6e-10*0.9          cgop3v = 2.9e-10*1.1          cjn3iso = 8.278441e-04*0.9

+cjp3iso = 1.446915e-03*1.1    cjswn3iso = 2.071174e-10*0.9  cjswp3iso = 3.226717e-11*1.1

+cjswgn3iso = 1.414663e-10*0.9 cjswgp3iso = 1.614054e-10*1.1 cgon3iso = 1.6e-10*0.9

+cgop3iso = 1.94e-10*1.1       cjn3 = 8.278441e-04*0.9       cjp3 = 1.446915e-03*1.1

+cjswn3 = 2.071174e-10*0.9     cjswp3 = 3.226717e-11*1.1     cjswgn3 = 1.414663e-10*0.9

+cjswgp3 = 1.614054e-10*1.1    cgon3 = 1.6e-10*0.9           cgop3 = 1.94e-10*1.1

include "c035logic3v5v.scs" section=mos

endsection fs

//  ************************************************************************

//

//  **************************************************************

//  *                                                            *

//  *           5.0V NORMAL and isolation DEVICES LIB            *

//  *                                                            *

//  **************************************************************

//  *************** CORNER_LIB OF TYPICAL MODEL ****************************

section tt_hv

parameters toxnl=1.47e-08

parameters toxpl=1.47e-08

parameters dxlnl=0

parameters dxlpl=0

parameters dxwnl=0

parameters dxwpl=0

parameters dvtnl=0

parameters dvtpl=0

parameters dvsatnl=1

parameters dvsatpl=1

parameters cjnl=7.5e-04

parameters cjpl=1.149006e-03

parameters cjswnl=1.01e-09

parameters cjswpl=2.212296e-10

parameters cjswgnl=1.19e-10

parameters cjswgpl=1.101914e-10

parameters cgonl=1.0965e-10

parameters cgopl=1.2067e-10

parameters dtoxnliso=0

parameters dtoxpliso= 0

parameters dxlnliso=0

parameters dxlpliso=0

parameters dxwnliso=0

parameters dxwpliso=0

parameters dvtnliso=0

parameters dvtpliso=0

parameters dvsatnliso=1

parameters dvsatpliso=1

parameters cjnliso=7.5e-04

parameters cjpliso=1.149006e-03

parameters cjswnliso=1.01e-09

parameters cjswpliso=2.212296e-10

parameters cjswgnliso=1.19e-10

parameters cjswgpliso=1.101914e-10

parameters cgonliso=1.0965e-10

parameters cgopliso=1.2067e-10

parameters dtoxn5l = 0

parameters dtoxp5l = 0

parameters dxln5l = 0

parameters dxlp5l = 0

parameters dxwn5l = 0

parameters dxwp5l = 0

parameters dvtn5l = 0

parameters dvtp5l = 0

parameters dvsatn5l = 1

parameters dvsatp5l = 1

parameters cjn5l = 7.5e-4

parameters cjp5l = 1.149006e-3

parameters cjswn5l = 1.01e-9

parameters cjswp5l = 2.212296e-10

parameters cjswgn5l = 1.19e-10

parameters cjswgp5l = 1.101914e-10

parameters cgon5l = 1.0965e-10

parameters cgop5l = 1.2067e-10

include "c035logic3v5v.scs" section=mos_hv

endsection tt_hv

//  ************** CORNER_LIB OF  SNSP   MODEL *****************************

section ss_hv

parameters toxnl=1.57e-08

parameters toxpl=1.57e-08

parameters dxlnl=4.0e-08

parameters dxlpl=4.0e-08

parameters dxwnl=-1.1e-08

parameters dxwpl=-1.1e-08

parameters dvtnl=0.168

parameters dvtpl=-0.18

parameters dvsatnl=0.87

parameters dvsatpl=0.77

parameters cjnl=7.5e-04 * 1.1

parameters cjpl=1.149006e-03 * 1.1

parameters cjswnl=1.01e-09 * 1.1

parameters cjswpl=2.212296e-10 * 1.1

parameters cjswgnl=1.639601e-10 * 1.1

parameters cjswgpl=1.101914e-10 * 1.1

parameters cgonl=1.0965e-10 * 1.1

parameters cgopl=1.2067e-10 * 1.1

parameters dtoxnliso= 15e-10

parameters dtoxpliso= 15e-10

parameters dxlnliso= 0.06e-6

parameters dxlpliso= 0.05e-6

parameters dxwnliso= -0.06e-6

parameters dxwpliso= -0.06e-6

parameters dvtnliso= 1.68E-1

parameters dvtpliso= -1.49E-1

parameters dvsatnliso= 9.24E-1

parameters dvsatpliso= 8.952E-1

parameters cjnliso=7.5e-04 * 1.1

parameters cjpliso=1.149006e-03 * 1.1

parameters cjswnliso=1.01e-09 * 1.1

parameters cjswpliso=2.212296e-10 * 1.1

parameters cjswgnliso=1.639601e-10 * 1.1

parameters cjswgpliso=1.101914e-10 * 1.1

parameters cgonliso=1.0965e-10 * 1.1

parameters cgopliso=1.2067e-10 * 1.1

parameters

+dtoxn5l = 6.29e-10

+dtoxp5l = 3.82e-10            dxln5l = 5e-8                 dxlp5l = 5e-8

+dxwn5l = -6e-8                dxwp5l = -6e-8                dvtn5l = 0.152

+dvtp5l = -0.151               dvsatn5l = 0.766              dvsatp5l = 0.8

+cjn5l = 7.5e-04*1.1           cjp5l = 1.149006e-03*1.1

+cjswn5l = 1.01e-09*1.1        cjswp5l = 2.212296e-10*1.1    cjswgn5l = 1.639601e-10*1.1

+cjswgp5l = 1.101914e-10*1.1   cgon5l = 1.0965e-10*1.1       cgop5l = 1.2067e-10*1.1

include "c035logic3v5v.scs" section=mos_hv

endsection ss_hv

//  ************** CORNER_LIB OF  FNFP   MODEL ****************************

section ff_hv

parameters toxnl=1.37e-08

parameters toxpl=1.37e-08

parameters dxlnl=-4.0e-08

parameters dxlpl=-4.0e-08

parameters dxwnl=1.1e-08

parameters dxwpl=1.1e-08

parameters dvtnl=-0.1489

parameters dvtpl=0.115

parameters dvsatnl=1.081

parameters dvsatpl=1.2

parameters cjnl=7.5e-04 * 0.9

parameters cjpl=1.149006e-03 * 0.9

parameters cjswnl=1.01e-09 * 0.9

parameters cjswpl=2.212296e-10 * 0.9

parameters cjswgnl=1.639601e-10 * 0.9

parameters cjswgpl=1.101914e-10 * 0.9

parameters cgonl=1.0965e-10 * 0.9

parameters cgopl=1.2067e-10 * 0.9

parameters dtoxnliso= -15e-10

parameters dtoxpliso= -15e-10

parameters dxlnliso= -0.06e-6

parameters dxlpliso= -0.05e-6

parameters dxwnliso= 0.06e-6

parameters dxwpliso= 0.06e-6

parameters dvtnliso= -1.739E-1

parameters dvtpliso= 1.48E-1

parameters dvsatnliso= 9.99E-1

parameters dvsatpliso= 9.69E-1

parameters cjnliso=7.5e-04 * 0.9

parameters cjpliso=1.149006e-03 * 0.9

parameters cjswnliso=1.01e-09 * 0.9

parameters cjswpliso=2.212296e-10 * 0.9

parameters cjswgnliso=1.639601e-10 * 0.9

parameters cjswgpliso=1.101914e-10 * 0.9

parameters cgonliso=1.0965e-10 * 0.9

parameters cgopliso=1.2067e-10 * 0.9

parameters

+dtoxn5l = -6.29e-10

+dtoxp5l = -3.82e-10           dxln5l = -5e-8                dxlp5l = -5e-8

+dxwn5l = 6e-8                 dxwp5l = 6e-8                 dvtn5l = -0.152

+dvtp5l = 0.15                 dvsatn5l = 1.2                dvsatp5l = 1.15

+cjn5l = 7.5e-04*0.9           cjp5l = 1.149006e-03*0.9

+cjswn5l = 1.01e-09*0.9        cjswp5l = 2.212296e-10*0.9    cjswgn5l = 1.639601e-10*0.9

+cjswgp5l = 1.101914e-10*0.9   cgon5l = 1.0965e-10*0.9       cgop5l = 1.2067e-10*0.9

include "c035logic3v5v.scs" section=mos_hv

endsection ff_hv

//  ************** CORNER_LIB OF  SNFP   MODEL ****************************

section sf_hv

parameters toxnl=1.57e-08

parameters toxpl=1.37e-08

parameters dxlnl=4.0e-08

parameters dxlpl=-4.0e-08

parameters dxwnl=-1.1e-08

parameters dxwpl=1.1e-08

parameters dvtnl=0.168

parameters dvtpl=0.115

parameters dvsatnl=0.87

parameters dvsatpl=1.2

parameters cjnl=7.5e-04 * 1.1

parameters cjpl=1.149006e-03 * 0.9

parameters cjswnl=1.01e-09 * 1.1

parameters cjswpl=2.212296e-10 * 0.9

parameters cjswgnl=1.639601e-10 * 1.1

parameters cjswgpl=1.101914e-10 * 0.9

parameters cgonl=1.0965e-10 * 1.1

parameters cgopl=1.2067e-10 * 0.9

parameters dtoxnliso= 15e-10

parameters dtoxpliso= -15e-10

parameters dxlnliso= 0.06e-6

parameters dxlpliso= -0.05e-6

parameters dxwnliso= -0.06e-6

parameters dxwpliso=0.06e-6

parameters dvtnliso= 1.68E-1

parameters dvtpliso= 1.48E-1

parameters dvsatnliso= 9.24E-1

parameters dvsatpliso= 9.69E-1

parameters cjnliso=7.5e-04 * 1.1

parameters cjpliso=1.149006e-03 * 0.9

parameters cjswnliso=1.01e-09 * 1.1

parameters cjswpliso=2.212296e-10 * 0.9

parameters cjswgnliso=1.639601e-10 * 1.1

parameters cjswgpliso=1.101914e-10 * 0.9

parameters cgonliso=1.0965e-10 * 1.1

parameters cgopliso=1.2067e-10 * 0.9

parameters

+dtoxn5l = 6.29e-10

+dtoxp5l = -3.82e-10           dxln5l = 5e-8                 dxlp5l = -5e-8

+dxwn5l = -6e-8                dxwp5l = 6e-8                 dvtn5l = 0.152

+dvtp5l = 0.15                 dvsatn5l = 0.766              dvsatp5l = 1.15

+cjn5l = 7.5e-04*1.1           cjp5l = 1.149006e-03*0.9

+cjswn5l = 1.01e-09*1.1        cjswp5l = 2.212296e-10*0.9    cjswgn5l = 1.639601e-10*1.1

+cjswgp5l = 1.101914e-10*0.9   cgon5l = 1.0965e-10*1.1       cgop5l = 1.2067e-10*0.9

include "c035logic3v5v.scs" section=mos_hv

endsection sf_hv

//  ************** CORNER_LIB OF  FNSP   MODEL ****************************

section fs_hv

parameters toxnl=1.37e-08

parameters toxpl=1.57e-08

parameters dxlnl=-4.0e-08

parameters dxlpl=4.0e-08

parameters dxwnl=1.1e-08

parameters dxwpl=-1.1e-08

parameters dvtnl=-0.1489

parameters dvtpl=-0.18

parameters dvsatnl=1.081

parameters dvsatpl=0.77

parameters cjnl=7.5e-04 * 0.9

parameters cjpl=1.149006e-03 * 1.1

parameters cjswnl=1.01e-09 * 0.9

parameters cjswpl=2.212296e-10 * 1.1

parameters cjswgnl=1.639601e-10 * 0.9

parameters cjswgpl=1.101914e-10 * 1.1

parameters cgonl=1.0965e-10 * 0.9

parameters cgopl=1.2067e-10 * 1.1

parameters dtoxnliso= -15e-10

parameters dtoxpliso= 15e-10

parameters dxlnliso= -0.06e-6

parameters dxlpliso= 0.05e-6

parameters dxwnliso= 0.06e-6

parameters dxwpliso= -0.06e-6

parameters dvtnliso= -1.739E-1

parameters dvtpliso= -1.49E-1

parameters dvsatnliso= 9.99E-1

parameters dvsatpliso= 8.952E-1

parameters cjnliso=7.5e-04 * 0.9

parameters cjpliso=1.149006e-03 * 1.1

parameters cjswnliso=1.01e-09 * 0.9

parameters cjswpliso=2.212296e-10 * 1.1

parameters cjswgnliso=1.639601e-10 * 0.9

parameters cjswgpliso=1.101914e-10 * 1.1

parameters cgonliso=1.0965e-10 * 0.9

parameters cgopliso=1.2067e-10 * 1.1

parameters

+dtoxn5l = -6.29e-10

+dtoxp5l = 3.82e-10            dxln5l = -5e-8                dxlp5l = 5e-8

+dxwn5l = 6e-8                 dxwp5l = -6e-8                dvtn5l = -0.152

+dvtp5l = -0.151               dvsatn5l = 1.2                dvsatp5l = 0.8

+cjn5l = 7.5e-04*0.9           cjp5l = 1.149006e-03*1.1

+cjswn5l = 1.01e-09*0.9        cjswp5l = 2.212296e-10*1.1    cjswgn5l = 1.639601e-10*0.9

+cjswgp5l = 1.101914e-10*1.1   cgon5l = 1.0965e-10*0.9       cgop5l = 1.2067e-10*1.1

include "c035logic3v5v.scs" section=mos_hv

endsection fs_hv

//  ************************************************************************

//  **************************************************************

//                                                               *

//                      Resistor LIB                             *

//                                                               *

//  **************************************************************

//  *************** CORNER_LIB OF TYPICAL MODEL ****************************

section t_res

parameters drsh=1

include "c035logic3v5v.scs" section=res

endsection t_res

//  ************** CORNER_LIB OF  Slow   MODEL *****************************

section s_res

parameters drsh=1.2

include "c035logic3v5v.scs" section=res

endsection s_res

//  ************** CORNER_LIB OF  Fast   MODEL *****************************

section f_res

parameters drsh=0.8

include "c035logic3v5v.scs" section=res

endsection f_res

//  ************************************************************************

//  **************************************************************

//                                                               *

//                      BIPOLAR LIB                              *

//                                                               *

//  **************************************************************

//  ************ CORNER_LIB OF TYPICAL MODEL *******************************

section t_bip

parameters isa=1

parameters bfa=1

parameters nfa=1

parameters rba=1

parameters rea=1

parameters rca=1

parameters rbma=1

parameters cjea=1

parameters cjca=1

include "c035logic3v5v.scs" section=bip

endsection t_bip

//  ************ CORNER_LIB OF   SS   MODEL ********************************

section s_bip

parameters isa=0.70

parameters bfa=0.85

parameters nfa=1.005

parameters rba=1.2

parameters rea=1.2

parameters rca=1.2

parameters rbma=1.2

parameters cjea=1.05

parameters cjca=1.05

include "c035logic3v5v.scs" section=bip

endsection s_bip

//  ************ CORNER_LIB OF   FF   MODEL ********************************

section f_bip

parameters isa=1.30

parameters bfa=1.15

parameters nfa=0.995

parameters rba=0.8

parameters rea=0.8

parameters rca=0.8

parameters rbma=0.8

parameters cjea=0.95

parameters cjca=0.95

include "c035logic3v5v.scs" section=bip

endsection f_bip

//  ***********************************************************************

//  **************************************************************

//                                                               *

//                      POP capacitor LIB                        *

//                                                               *

//  **************************************************************

//  *************** CORNER_LIB OF TYPICAL MODEL ****************************

section t_cap

parameters ducap=1

include "c035logic3v5v.scs" section=cap

endsection t_cap

//  ************** CORNER_LIB OF  Slow   MODEL *****************************

section s_cap

parameters ducap=1.2

include "c035logic3v5v.scs" section=cap

endsection s_cap

//  ************** CORNER_LIB OF  Fast   MODEL *****************************

section f_cap

parameters ducap=0.8

include "c035logic3v5v.scs" section=cap

endsection f_cap

//  ************************************************************************

//  ************************************************************************

//

//  ***********************************************************************

//

//  *************************************************************

//  *                                                           *

//  *                     3.3V DEVICES MODEL                    *

//  *                                                           *

//  *************************************************************

//

section mos

//  *************************************************************

//  *                                                           *

//  *               3.3V NORMAL NMOS DEVICES MODEL              *

//  *                                                           *

//  *************************************************************

model n3 bsim3v3    {

1: type=n

+ version=3.2               binunit=2                 binflag=0

+ mobmod=1                  capmod=3                  nqsmod=0

+ lmin=1e-6                 lmax=2.001e-4              wmin=1e-6

+ wmax=2.001e-4              tox=toxn3v                toxm=toxn3v

+ xj=1e-7                   nch=2.998498e17           lnch=4.894158e6

+ wnch=0                    pnch=5.820766e-11         wint=0

+ wl=0                      wln=1                     ww=0

+ wwn=1                     wwl=0                     lint=0

+ ll=0                      lln=1                     lw=0

+ lwn=1                     lwl=0                     llc=0

+ lwc=0                     lwlc=0                    wlc=0

+ wwc=0                     wwlc=0                    dwg=0

+ dwb=0                     wmlt=1                    lmlt=1

+ xl= - 1.3e-8 + dxln3v     xw= - 1.8e-8 + dxwn3v

+ vth0=(0.519099 + dvtn3v) + 0.022969                 lvth0=1.639087e-7

+ wvth0=1.230145e-7         pvth0=-1.216396e-13       vfb=-0.930818

+ k1=0.804455               lk1=-7.867119e-9          wk1=-1.85379e-8

+ pk1=-5.409265e-14         k2=-3.20237e-2            lk2=0

+ wk2=4.376778e-9           pk2=-6.310887e-30         k3=0

+ k3b=0                     w0=0                      nlx=0

+ dvt0=0                    dvt1=0                    dvt2=0

+ dvt0w=0                   dvt1w=0                   dvt2w=0

+ ngate=1e23                u0=3.03793e-2             lu0=4.273151e-10

+ wu0=-1.593075e-10         pu0=1.35778e-15           ua=-1.66475e-9

+ lua=-3.769547e-16         wua=4.983875e-16          pua=8.627063e-23

+ ub=3.127827e-18           lub=3.813064e-25          wub=-2.423251e-25

+ pub=-3.11459e-31          uc=6.245481e-11           luc=1.821614e-17

+ wuc=8.103617e-18          puc=-2.81925e-23

+ vsat=2.398133e5 * dvsatn3v                          lvsat=-0.136294

+ wvsat=1.122983e-3         pvsat=-2.246415e-8        a0=1.071553

+ la0=-3.060072e-8          wa0=8.835558e-8           pa0=-4.545783e-13

+ ags=0.225925              lags=-1.547661e-7         wags=-4.017106e-8

+ pags=3.985692e-14         b0=0                      b1=0

+ keta=-2.74858e-2          lketa=1.353486e-8         wketa=2.874436e-8

+ pketa=-2.758279e-14       a1=0                      a2=0.99

+ rdsw=549                  prwb=0                    prwg=0

+ wr=1                      voff=-0.118429            nfactor=1

+ eta0=1.111358e-4          leta0=6.906157e-10        weta0=-2.413021e-11

+ peta0=2.422673e-17        etab=-2.806194e-7         letab=5.61351e-12

+ wetab=5.614913e-12        petab=-1.123207e-16       dsub=0

+ cit=0                     cdsc=0                    cdscb=0

+ cdscd=0                   pclm=1.3                  pdiblc1=1.117587e-8

+ pdiblc2=1.000004e-5       pdiblcb=0                 drout=0

+ pscbe1=4.24e8             pscbe2=4.5e-6             pvag=0

+ delta=0.01                alpha0=0                  alpha1=0

+ beta0=30                  elm=5                     xpart=0

+ cgso=cgon3v               cgdo=cgon3v               cgbo=5.0022e-14

+ ckappa=0.6                cf=0                      clc=5e-8

+ cle=0.996                 dlc=8.5e-9                dwc=0

+ vfbcv=-1                  noff=1                    voffcv=0

+ acde=0.5                  moin=15                   cgsl=9e-11

+ cgdl=9e-11                tlevc=1                   ldif=1e-7

+ hdif=4.5e-7               rsh=85                    rd=0

+ rs=0                      rsc=0                     rdc=0

+ cj=cjn3v                  mj=0.269967               mjsw=8.18799e-2

+ cjsw=cjswn3v              cjswg=cjswgn3v            mjswg=0.185718

+ js=1e-6                   jsw=1e-12                 pbsw=0.112146

+ pb=0.632866               pbswg=0.137237            tnom=25

+ ute=-0.756816             lute=1.684053e-8          wute=1.363854e-7

+ pute=-3.369622e-13        kt1=-0.340714             lkt1=-7.280506e-8

+ wkt1=8.369716e-8          pkt1=-2.257512e-13        kt1l=4.839549e-10

+ lkt1l=-9.674211e-15       wkt1l=-9.676629e-15       pkt1l=1.935713e-19

+ kt2=-3.66291e-2           lkt2=1.299189e-8          wkt2=4.176153e-8

+ pkt2=-5.024668e-14        ua1=5.012117e-9           lua1=-3.646644e-16

+ wua1=1.756006e-16         pua1=-1.06616e-21         ub1=-6.006429e-18

+ lub1=-6.941543e-26        wub1=-6.944279e-26        pub1=1.348747e-30

+ uc1=-9.52e-11             luc1=1.169324e-23         wuc1=1.174881e-23

+ puc1=-2.246415e-29        at=1.081223e5             lat=-9.13027e-2

+ wat=-7.41269e-2           pat=5.868758e-8           prt=0.1

+ lprt=0                    wprt=0                    pprt=0

+ nj=1                      xti=3                     tpb=1.716697e-3

+ tpbsw=7.957164e-4         tpbswg=6.83e-4            tcj=9.707176e-4

+ tcjsw=1.321796e-3         tcjswg=1.72e-3

2: type=n

+ version=3.2               binunit=2                 binflag=0

+ mobmod=1                  capmod=3                  nqsmod=0

+ lmin=6e-7                 lmax=1.001e-6              wmin=1e-6

+ wmax=2.001e-4              tox=toxn3v                toxm=toxn3v

+ xj=1e-7                   nch=2.998546e17           wint=0

+ wl=0                      wln=1                     ww=0

+ wwn=1                     wwl=0                     lint=0

+ ll=0                      lln=1                     lw=0

+ lwn=1                     lwl=0                     llc=0

+ lwc=0                     lwlc=0                    wlc=0

+ wwc=0                     wwlc=0                    dwg=0

+ dwb=0                     wmlt=1                    lmlt=1

+ xl= - 1.3e-8 + dxln3v     xw= - 1.8e-8 + dxwn3v     vth0=0.583226 + dvtn3v

+ lvth0=1.244903e-7         wvth0=7.370434e-8         pvth0=-7.399916e-14

+ vfb=-0.930818             k1=0.955123               lk1=-1.59138e-7

+ wk1=-1.742206e-7          pk1=1.022128e-13          k2=-3.85737e-2

+ lk2=6.576185e-9           wk2=1.098571e-8           pk2=-6.635371e-15

+ k3=0                      k3b=0                     w0=0

+ nlx=0                     dvt0=0                    dvt1=0

+ dvt2=0                    dvt0w=0                   dvt1w=0

+ dvt2w=0                   ngate=1e23                u0=2.25932e-2

+ lu0=8.244521e-9           wu0=4.117739e-9           pu0=-2.936374e-15

+ ua=-1.941456e-9           lua=-9.914157e-17         wua=7.621248e-16

+ pua=-1.785217e-22         ub=3.103724e-18           lub=4.055066e-25

+ wub=-6.287568e-25         pub=7.651847e-32          uc=6.386502e-11

+ luc=1.680029e-17          wuc=-3.563647e-17         puc=1.572255e-23

+ vsat=1.562809e5 * dvsatn3v                          lvsat=-4.33916e-2

+ wvsat=-5.51419e-2         pvsat=3.402582e-8         a0=1.853068

+ la0=-8.11989e-7           wa0=-9.226574e-7          pa0=5.572851e-13

+ ags=0.143331              lags=-7.207592e-8         wags=-6.13456e-10

+ pags=3.705274e-16         b0=0                      b1=0

+ keta=1.72296e-2           lketa=-3.089976e-8        wketa=-3.344177e-9

+ pketa=4.18275e-15         a1=0                      a2=0.99

+ rdsw=549                  prwb=0                    prwg=0

+ wr=1                      voff=-0.118429            nfactor=1

+ eta0=-1.867257e-4         leta0=9.896686e-10        weta0=-2.628974e-10

+ peta0=2.63949e-16         etab=6.800277e-4          letab=-6.774161e-10

+ wetab=-2.522679e-10       petab=1.465936e-16        dsub=0

+ cit=0                     cdsc=0                    cdscb=0

+ cdscd=0                   pclm=1.3                  pdiblc1=1.117587e-8

+ pdiblc2=1.000004e-5       pdiblcb=0                 drout=0

+ pscbe1=4.24e8             pscbe2=4.5e-6             pvag=0

+ delta=0.01                alpha0=0                  alpha1=0

+ beta0=30                  elm=5                     xpart=0

+ cgso=cgon3v               cgdo=cgon3v               cgbo=5.0022e-14

+ ckappa=0.6                cf=0                      clc=5e-8

+ cle=0.996                 dlc=8.5e-9                dwc=0

+ vfbcv=-1                  noff=1                    voffcv=0

+ acde=0.5                  moin=15                   cgsl=9e-11

+ cgdl=9e-11                tlevc=1                   ldif=1e-7

+ hdif=4.5e-7               rsh=85                    rd=0

+ rs=0                      rsc=0                     rdc=0

+ cj=cjn3v                  mj=0.269967               mjsw=8.18799e-2

+ cjsw=cjswn3v              cjswg=cjswgn3v            mjswg=0.185718

+ js=1e-6                   jsw=1e-12                 pbsw=0.112146

+ pb=0.632866               pbswg=0.137237            tnom=25

+ ute=-0.441882             lute=-2.993531e-7         wute=-5.000782e-7

+ pute=3.020472e-13         kt1=-0.142149             lkt1=-2.721645e-7

+ wkt1=-2.815661e-7         pkt1=1.409731e-13         kt1l=-2.832017e-7

+ lkt1l=2.751462e-13        wkt1l=4.596399e-13        pkt1l=-2.776225e-19

+ kt2=3.848364e-3           lkt2=-2.764745e-8         wkt2=-3.60701e-8

+ pkt2=2.789628e-14         ua1=6.679712e-9           lua1=-2.038929e-15

+ wua1=-2.934633e-15        pua1=2.056515e-21         ub1=-8.820234e-18

+ lub1=2.755645e-24         wub1=4.043299e-24         pub1=-2.780446e-30

+ uc1=-1.371797e-10         luc1=4.214767e-17         wuc1=-2.171855e-17

+ puc1=2.180541e-23         at=2.211289e4             lat=-4.949304e-3

+ wat=-2.06471e-2           pat=4.993848e-9           prt=0.1

+ nj=1                      xti=3                     tpb=1.716697e-3

+ tpbsw=7.957164e-4         tpbswg=6.83e-4            tcj=9.707176e-4

+ tcjsw=1.321796e-3         tcjswg=1.72e-3

3: type=n

+ version=3.2               binunit=2                 binflag=0

+ mobmod=1                  capmod=3                  nqsmod=0

+ lmin=3.5e-7               lmax=6.01e-7              wmin=1e-6

+ wmax=2.001e-4              tox=toxn3v                toxm=toxn3v

+ xj=1e-7                   nch=2.998546e17           wint=0

+ wl=0                      wln=1                     ww=0

+ wwn=1                     wwl=0                     lint=0

+ ll=0                      lln=1                     lw=0

+ lwn=1                     lwl=0                     llc=0

+ lwc=0                     lwlc=0                    wlc=0

+ wwc=0                     wwlc=0                    dwg=0

+ dwb=0                     wmlt=1                    lmlt=1

+ xl= - 1.3e-8 + dxln3v     xw= - 1.8e-8 + dxwn3v

+ vth0=(0.972955 + dvtn3v) - 0.025062                 lvth0=-9.631098e-8

+ wvth0=-8.212708e-8        pvth0=2.065409e-14        vfb=-0.930818

+ k1=0.889036               lk1=-1.243821e-7          wk1=2.292141e-7

+ pk1=-1.363942e-13         k2=-3.29096e-2            lk2=3.242704e-9

+ wk2=5.40982e-9            pk2=-3.353616e-15         k3=0

+ k3b=0                     w0=0                      nlx=0

+ dvt0=0                    dvt1=0                    dvt2=0

+ dvt0w=0                   dvt1w=0                   dvt2w=0

+ ngate=1e23                u0=1.91889e-2             lu0=1.035129e-8

+ wu0=1.654418e-8           pu0=-1.049161e-14         ua=-3.54514e-9

+ lua=8.580109e-16          wua=2.482192e-15          pua=-1.206176e-21

+ ub=3.596579e-18           lub=1.141401e-25          wub=-9.441354e-25

+ pub=2.608029e-31          uc=7.944985e-11           luc=5.577905e-18

+ wuc=1.113746e-16          puc=-7.129558e-23

+ vsat=(8.702134e4 * dvsatn3v) + 2.031976e3           lvsat=-2.751646e-3

+ wvsat=7.053933e-3         pvsat=-3.574397e-9        a0=0.440904

+ la0=3.980527e-8           wa0=6.659082e-8           pa0=-3.908876e-14

+ ags=-1.633695             lags=9.978091e-7          wags=1.667153e-6

+ pags=-1.003583e-12        b0=0                      b1=0

+ keta=1.23237e-2           lketa=-2.721154e-8        wketa=-4.090775e-8

+ pketa=2.615911e-14        a1=0                      a2=0.99

+ rdsw=535.453481           lrdsw=7.951807e-6         wrdsw=1.330268e-5

+ prdsw=-7.808674e-12       prwb=0                    prwg=0

+ wr=1                      voff=-0.173206            lvoff=3.308515e-8

+ wvoff=5.526973e-8         pvoff=-3.338292e-14       nfactor=1

+ eta0=-3.204164e-3         leta0=3.068327e-9         weta0=-1.55107e-8

+ peta0=9.222104e-15        etab=-9.24473e-4          letab=2.917024e-10

+ wetab=7.049045e-10        petab=-4.315385e-16       dsub=0

+ cit=0                     cdsc=0                    cdscb=0

+ cdscd=0                   pclm=1.3                  pdiblc1=1.117587e-8

+ pdiblc2=1.000004e-5       pdiblcb=0                 drout=0

+ pscbe1=4.24e8             pscbe2=4.5e-6             pvag=0

+ delta=0.01                alpha0=0                  alpha1=0

+ beta0=30                  elm=5                     xpart=0

+ cgso=cgon3v               cgdo=cgon3v               cgbo=5.0022e-14

+ ckappa=0.6                cf=0                      clc=5e-8

+ cle=0.996                 dlc=8.5e-9                dwc=0

+ vfbcv=-1                  noff=1                    voffcv=0

+ acde=0.5                  moin=15                   cgsl=9e-11

+ cgdl=9e-11                tlevc=1                   ldif=1e-7

+ hdif=4.5e-7               rsh=85                    rd=0

+ rs=0                      rsc=0                     rdc=0

+ cj=cjn3v                  mj=0.269967               mjsw=8.18799e-2

+ cjsw=cjswn3v              cjswg=cjswgn3v            mjswg=0.185718

+ js=1e-6                   jsw=1e-12                 pbsw=0.112146

+ pb=0.632866               pbswg=0.137237            tnom=25

+ ute=-0.9375               kt1=-0.416397             lkt1=-1.065187e-7

+ wkt1=-7.947047e-8         pkt1=1.890736e-14         kt1l=1.723383e-7

+ kt2=-0.071489             lkt2=1.785635e-8          wkt2=1.417224e-8

+ pkt2=-2.450097e-15        ua1=4.195002e-9           lua1=-5.381647e-16

+ wua1=1.469259e-15         pua1=-6.034361e-22        ub1=-6.017172e-18

+ lub1=1.062595e-24         wub1=-2.075388e-24        pub1=9.152416e-31

+ uc1=-1.560954e-10         luc1=5.357274e-17         wuc1=3.474963e-17

+ puc1=-1.230137e-23        at=-5.223643e3            lat=0.011562

+ wat=-1.18525e-2           pat=-3.180762e-10         prt=0.1

+ lprt=0                    wprt=0                    pprt=0

+ nj=1                      xti=3                     tpb=1.716697e-3

+ tpbsw=7.957164e-4         tpbswg=6.83e-4            tcj=9.707176e-4

+ tcjsw=1.321796e-3         tcjswg=1.72e-3

4: type=n

+ version=3.2               binunit=2                 binflag=0

+ mobmod=1                  capmod=3                  nqsmod=0

+ lmin=3.5e-7               lmax=2.001e-4              wmin=6e-7

+ wmax=1.001e-6              tox=toxn3v                toxm=toxn3v

+ xj=1e-7                   nch=2.998428e17           lnch=4.208911e6

+ wnch=7.240753e6           pnch=-2.563227            wint=0

+ wl=0                      wln=1                     ww=0

+ wwn=1                     wwl=0                     lint=0

+ ll=0                      lln=1                     lw=0

+ lwn=1                     lwl=0                     llc=0

+ lwc=0                     lwlc=0                    wlc=0

+ wwc=0                     wwlc=0                    dwg=0

+ dwb=0                     wmlt=1                    lmlt=1

+ xl= - 1.3e-8 + dxln3v     xw= - 1.8e-8 + dxwn3v     vth0=0.729759 + dvtn3v

+ lvth0=-2.763284e-8        wvth0=-6.099123e-8        pvth0=2.159089e-14

+ vfb=-0.930818             k1=0.852323               lk1=-1.680378e-7

+ wk1=-6.256934e-8          pk1=2.214954e-14          k2=-0.027686

+ k3=0                      k3b=0                     w0=0

+ nlx=0                     dvt0=0                    dvt1=0

+ dvt2=0                    dvt0w=0                   dvt1w=0

+ dvt2w=0                   ngate=1e23                u0=3.02164e-2

+ lu0=1.872481e-9           wu0=-1.654361e-24         pu0=3.155444e-30

+ ua=-1.993718e-9           lua=-6.388493e-17         wua=8.313341e-16

+ pua=-2.499712e-22         ub=3.986056e-18           lub=-7.949465e-27

+ wub=-1.119678e-24         pub=3.093556e-31          uc=1.254345e-10

+ luc=-4.294888e-17         wuc=-5.473572e-17         puc=1.937644e-23

+ vsat=2.450917e5 * dvsatn3v                          lvsat=-6.18469e-2

+ wvsat=-1.439105e-4        pvsat=2.878786e-9         a0=1.151308

+ la0=-2.274777e-7          wa0=-6.121137e-23         pa0=5.04871e-29

+ ags=0.18281               lags=-5.621888e-8         wags=6.100457e-24

+ pags=3.155444e-30         b0=0                      b1=0

+ keta=9.537076e-3          lketa=-2.763204e-8        wketa=-8.030415e-9

+ pketa=1.620851e-14        a1=0                      a2=0.99

+ rdsw=549                  prwb=0                    prwg=0

+ wr=1                      voff=-0.118429            nfactor=1

+ eta0=-2.08021e-4          leta0=5.96373e-9          weta0=4.215623e-11

+ peta0=-1.804662e-16       etab=7.643067e-6          letab=-1.528919e-10

+ wetab=3.376362e-12        petab=-6.754075e-17       dsub=0

+ cit=0                     cdsc=0                    cdscb=0

+ cdscd=0                   pclm=1.3                  pdiblc1=1.117587e-8

+ pdiblc2=1.000004e-5       pdiblcb=0                 drout=0

+ pscbe1=4.24e8             pscbe2=4.5e-6             pvag=0

+ delta=0.01                alpha0=0                  alpha1=0

+ beta0=30                  elm=5                     xpart=0

+ cgso=cgon3v               cgdo=cgon3v               cgbo=5.0022e-14

+ ckappa=0.6                cf=0                      clc=5e-8

+ cle=0.996                 dlc=8.5e-9                dwc=0

+ vfbcv=-1                  noff=1                    voffcv=0

+ acde=0.5                  moin=15                   cgsl=9e-11

+ cgdl=9e-11                tlevc=1                   ldif=1e-7

+ hdif=4.5e-7               rsh=85                    rd=0

+ rs=0                      rsc=0                     rdc=0

+ cj=cjn3v                  mj=0.269967               mjsw=8.18799e-2

+ cjsw=cjswn3v              cjswg=cjswgn3v            mjswg=0.185718

+ js=1e-6                   jsw=1e-12                 pbsw=0.112146

+ pb=0.632866               pbswg=0.137237            tnom=25

+ ute=-0.632095             lute=-1.081132e-7         wute=5.293956e-23

+ pute=-1.009742e-28        kt1=-0.263473             lkt1=-1.823331e-7

+ wkt1=-6.437228e-10        pkt1=1.287703e-14         kt1l=-3.104374e-9

+ lkt1l=6.210672e-14        wkt1l=0                   pkt1l=2.407412e-35

+ kt2=2.991396e-3           lkt2=-1.428203e-9         wkt2=2.329966e-10

+ pkt2=-4.660865e-15        ua1=5.210867e-9           lua1=-1.002039e-15

+ wua1=-4.80297e-17         pua1=3.888027e-23         ub1=-6.083123e-18

+ lub1=1.424737e-24         wub1=8.208155e-27         pub1=-1.641959e-31

+ uc1=-2.238645e-11         luc1=5.330205e-18         wuc1=-7.511283e-17

+ puc1=2.750768e-23         at=3.333336e4             lat=-6.668604e-3

+ wat=-5.53502e-6           pat=1.107225e-10          prt=0.1

+ nj=1                      xti=3                     tpb=1.716697e-3

+ tpbsw=7.957164e-4         tpbswg=6.83e-4            tcj=9.707176e-4

+ tcjsw=1.321796e-3         tcjswg=1.72e-3

5: type=n

+ version=3.2               binunit=2                 binflag=0

+ mobmod=1                  capmod=3                  nqsmod=0

+ lmin=3.5e-7               lmax=2.001e-4              wmin=4e-7

+ wmax=6.01e-7              tox=toxn3v                toxm=toxn3v

+ xj=1e-7                   nch=2.998546e17           wint=0

+ wl=0                      wln=1                     ww=0

+ wwn=1                     wwl=0                     lint=0

+ ll=0                      lln=1                     lw=0

+ lwn=1                     lwl=0                     llc=0

+ lwc=0                     lwlc=0                    wlc=0

+ wwc=0                     wwlc=0                    dwg=0

+ dwb=0                     wmlt=1                    lmlt=1

+ xl= - 1.3e-8 + dxln3v     xw= - 1.8e-8 + dxwn3v

+ vth0=(0.758141 + dvtn3v) - 0.036327                 lvth0=1.083216e-8

+ wvth0=-5.716812e-8        pvth0=-1.828337e-15       vfb=-0.930818

+ k1=0.910082               lk1=-1.73977e-7           wk1=-9.774469e-8

+ pk1=2.576652e-14          k2=-0.027686              k3=0

+ k3b=0                     w0=0                      nlx=0

+ dvt0=0                    dvt1=0                    dvt2=0

+ dvt0w=0                   dvt1w=0                   dvt2w=0

+ ngate=1e23                u0=3.06219e-2             lu0=-6.239242e-9

+ wu0=-2.469526e-10         pu0=4.940039e-15          ua=-1.108064e-9

+ lua=-3.624426e-16         wua=2.919709e-16          pua=-6.814963e-23

+ ub=3.030456e-18           lub=5.711415e-25          wub=-5.377178e-25

+ pub=-4.331077e-32         uc=9.435573e-11           luc=7.57597e-18

+ wuc=-3.580873e-17         puc=-1.139319e-23

+ vsat=(2.454633e5 * dvsatn3v) - 5.613445             lvsat=-6.81573e-2

+ wvsat=-3.66932e-4         pvsat=6.752099e-9         a0=1.151308

+ la0=-2.274777e-7          wa0=3.308722e-24          pa0=0

+ ags=0.189776              lags=-5.856637e-8         wags=-4.054115e-9

+ pags=1.366237e-15         b0=0                      b1=0

+ keta=5.556927e-4          lketa=-2.905436e-9        wketa=-2.560753e-9

+ pketa=1.150001e-15        a1=0                      a2=0.99

+ rdsw=549                  prwb=0                    prwg=0

+ wr=1                      voff=-0.116247            lvoff=-4.36504e-8

+ wvoff=-1.328889e-9        pvoff=2.658309e-14        nfactor=1

+ eta0=-3.949713e-5         leta0=3.680964e-9         weta0=-6.047484e-11

+ peta0=1.209739e-15        etab=3.059834e-5          letab=-6.120891e-10

+ wetab=-1.06034e-11        petab=2.121104e-16        dsub=0

+ cit=0                     cdsc=0                    cdscb=0

+ cdscd=0                   pclm=1.3                  pdiblc1=1.117587e-8

+ pdiblc2=1.000004e-5       pdiblcb=0                 drout=0

+ pscbe1=4.24e8             pscbe2=4.5e-6             pvag=0

+ delta=0.01                alpha0=0                  alpha1=0

+ beta0=30                  elm=5                     xpart=0

+ cgso=cgon3v               cgdo=cgon3v               cgbo=5.0022e-14

+ ckappa=0.6                cf=0                      clc=5e-8

+ cle=0.996                 dlc=8.5e-9                dwc=0

+ vfbcv=-1                  noff=1                    voffcv=0

+ acde=0.5                  moin=15                   cgsl=9e-11

+ cgdl=9e-11                tlevc=1                   ldif=1e-7

+ hdif=4.5e-7               rsh=85                    rd=0

+ rs=0                      rsc=0                     rdc=0

+ cj=cjn3v                  mj=0.269967               mjsw=8.18799e-2

+ cjsw=cjswn3v              cjswg=cjswgn3v            mjswg=0.185718

+ js=1e-6                   jsw=1e-12                 pbsw=0.112146

+ pb=0.632866               pbswg=0.137237            tnom=25

+ ute=-0.632095             lute=-1.081132e-7         wute=1.985233e-23

+ pute=0                    kt1=-0.265874             lkt1=-1.343038e-7

+ wkt1=8.184764e-10         pkt1=-1.63728e-14         kt1l=-3.104374e-9

+ lkt1l=6.210672e-14        wkt1l=0                   pkt1l=0

+ kt2=3.315224e-3           lkt2=-7.906041e-9         wkt2=3.57859e-11

+ pkt2=-7.158612e-16        ua1=5.72474e-9            lua1=-1.132439e-15

+ wua1=-3.609785e-16        pua1=1.182944e-22         ub1=-6.061304e-18

+ lub1=9.882776e-25         wub1=-5.079375e-27        pub1=1.016078e-31

+ uc1=-2.455461e-10         luc1=8.37389e-17          wuc1=6.07914e-17

+ puc1=-2.024321e-23        at=8.209081e4             lat=-4.11245e-2

+ wat=-2.96988e-2           pat=2.109434e-8           prt=0.1

+ nj=1                      xti=3                     tpb=1.716697e-3

+ tpbsw=7.957164e-4         tpbswg=6.83e-4            tcj=9.707176e-4

+ tcjsw=1.321796e-3         tcjswg=1.72e-3

}

//

//  *************************************************************

//  *                                                           *

//  *               3.3V PMOS DEVICES MODEL                     *

//  *                                                           *

//  *************************************************************

model p3 bsim3v3 type=p

+ version=3.2               binunit=2                 binflag=0

+ mobmod=1                  capmod=3                  nqsmod=0

+ lmin=3.5e-7               lmax=2.001e-4              wmin=4e-7

+ wmax=2.001e-4              tox=toxp3v                toxm=toxp3v

+ xj=1e-7                   nch=9.2981e16             wint=2.5e-8

+ wl=-4.11e-14              wln=1                     ww=-5.611755e-14

+ wwn=1                     wwl=-1.5e-22              lint=2.01e-8

+ ll=-2.2e-10               lln=0.3436                lw=4.03e-15

+ lwn=1                     lwl=-1.89e-17             llc=0

+ lwc=0                     lwlc=0                    wlc=0

+ wwc=0                     wwlc=0                    dwg=-5.019746e-8

+ dwb=1.324632e-10          wmlt=1                    lmlt=1

+ xl=4.00e-09 + dxlp3v      xw=9.00e-09 + dxwp3v

+ vth0=( - 0.754 + dvtp3v) - 0.03905                  pvth0=-5.456792e-14

+ vfb=-0.444151             k1=0.76225                k2=-0.049608

+ k3=-3.73e-8               k3b=2.439265              w0=-5.523068e-9

+ nlx=1.71e-7               dvt0=1.1643               dvt1=0.34587

+ dvt2=-0.060989            dvt0w=1.184075            dvt1w=2.812527e5

+ dvt2w=0.138288            ngate=1e23

+ u0=(0.014827 * du0p3v) - 1.4827e-4                  lu0=-4.968e-10

+ wu0=-5.720194e-9          pu0=-6.75e-16             ua=8.675e-11

+ lua=3.546e-18             ub=1.5059e-18             lub=-1.38e-25

+ uc=-1.09e-12              vsat=1.9984e5             pvsat=-3.62e-8

+ a0=1.0297                 ags=0.22875               b0=3.645787e-7

+ b1=7.2e-7                 keta=-5.291e-3            wketa=8.5e-9

+ pketa=8.0615e-16          a1=0                      a2=0.50984

+ rdsw=324.57               prdsw=6.75e-12            prwb=0.405

+ pprwb=2.37e-13            prwg=0.673417             pprwg=1.8725e-13

+ wr=1                      voff=-0.1776              lvoff=4.6008e-9

+ nfactor=2.001             lnfactor=-2.484e-7        eta0=0.02088

+ etab=-3e-3                dsub=0.208                cit=0

+ cdsc=8.796669e-4          cdscb=-6.638216e-5        cdscd=0

+ pclm=2.1107               pdiblc1=0.39              pdiblc2=8.6e-3

+ pdiblcb=-2.4996e-5        drout=1.1916              pscbe1=7e8

+ pscbe2=1.15e-5            pvag=0                    delta=0.01

+ alpha0=0                  alpha1=0                  beta0=30

+ elm=5                     xpart=0                   cgso=cgop3v

+ cgdo=cgop3v               cgbo=1e-13                ckappa=0.6

+ cf=0                      clc=0                     cle=1

+ dlc=0                     dwc=0                     vfbcv=-1

+ noff=1                    voffcv=0                  acde=1

+ moin=15                   cgsl=1e-10                cgdl=1e-10

+

+

+ ldif=1e-7                 hdif=4.5e-7               rsh=185

+ rd=0                      rs=0                      rsc=0

+ rdc=0                     cj=cjp3v                  mj=0.433828

+ mjsw=0.244471             cjsw=cjswp3v              cjswg=cjswgp3v

+ mjswg=0.249471            js=2.2e-7                 jsw=2.2e-13

+ pbsw=0.714128             pb=0.908111               pbswg=0.645099

+ tnom=25                   ute=-1.483                wute=-2.449158e-7

+ pute=7.725e-14            kt1=-0.58803              wkt1=-7.890177e-8

+ kt1l=1.1666e-8            kt2=-0.0695               lkt2=5.8e-9

+ ua1=4.438e-10             lua1=-3.249e-16           ub1=-1.5024e-18

+ lub1=-3e-27               uc1=-3.1069e-11           at=9.1228e3

+ pat=3.4e-9                prt=10                    nj=1

+ xti=3                     tpb=1.590593e-3           tpbsw=1.585214e-3

+ tpbswg=1.83e-3            tcj=9.147986e-4           tcjsw=6.63089e-4

+ tcjswg=9.16e-4

//  *************************************************************

//  *                                                           *

//  *               3.3V ISOLATED NMOS DEVICE MODEL            *

//  *                                                           *

//  *************************************************************

model n3iso bsim3v3

+version = 3.2                 binunit = 2                   mobmod = 1

+capmod = 3                    nqsmod = 0                    igcmod = 0

+igbmod = 0                    lmin = 4e-7                   lmax = 2.001e-4

+wmin = 4e-7                   wmax = 2.001e-4                tox = 9.2e-9+dtoxn3iso

+xj = 1e-7                     nch = 1e17                    wint = 1e-7

+wl = -1.427e-13               wln = 1                       ww = -4e-14

+wwn = 1                       wwl = 5.801e-20               lint = 0

+ll = 0                        lln = 1                       lw = -7e-14

+lwn = 1                       lwl = 0                       dwg = 0

+dwb = 0                       xl = -1.3e-8+dxln3iso         xw = 3e-9+dxwn3iso

+vth0 = 0.51+dvtn3iso          lvth0 = 7.12e-8               wvth0 = -5.6e-8

+pvth0 = -2e-15                k1 = 0.53                     lk1 = 0

+wk1 = 7.95e-8                 pk1 = -3e-14                  k2 = 0.03648

+lk2 = -1.3282e-8              wk2 = -4.0944e-8              pk2 = -3e-15

+k3 = 0                        k3b = -7.74                   w0 = 2.5e-6

+nlx = 1.74e-7                 dvt0 = 2.2                    dvt1 = 0.53

+dvt2 = -0.032                 dvt0w = 0                     dvt1w = 5.3e6

+dvt2w = -0.032                ngate = 1e23                  u0 = 0.0523*du0n3iso

+lu0 = 7.7e-9                  wu0 = -4e-9                   pu0 = 1.2845e-14

+ua = 1e-10                    lua = 3.68e-16                wua = -2.0025e-16

+pua = -1.955e-22              ub = 2.4828e-18               lub = -4.845e-25

+wub = -4.78125e-26            pub = 4e-32                   uc = 6.975e-11

+luc = -2.79e-17               wuc = -1.307813e-17           puc = -2.0925e-23

+vsat = 9.955e4                pvsat = 0                     a0 = 1.95

+la0 = -4.788e-7               wa0 = -1e-7                   pa0 = -2e-14

+ags = 0.26                    lags = -4.225e-8              wags = -1e-8

+pags = 7.02e-14               b0 = 0                        b1 = 0

+keta = -3.6e-3                lketa = -4.5e-9               wketa = -6e-10

+pketa = 1.323e-15             a1 = 0                        a2 = 1

+rdsw = 1.05e3                 prdsw = -1.775e-10            prwb = 0

+prwg = 0                      wr = 1                        voff = -0.14

+nfactor = 1                   eta0 = 0.08                   etab = -0.0175

+dsub = 0.56                   cit = 0                       cdsc = 2.4e-4

+cdscb = 0                     cdscd = 0                     pclm = 0.11725

+ppclm = 1.323562e-12          pdiblc1 = 4.857226e-17        pdiblc2 = 8.6e-3

+pdiblcb = 0                   drout = 0.56                  pdrout = 0

+pscbe1 = 4.24e8               pscbe2 = 1e-5                 pvag = 0

+delta = 0.01                  alpha0 = 8.2e-7               alpha1 = 4.27

+beta0 = 19.639                nigc = 1                      aigc = 0.012

+bigc = 2.8e-3                 cigc = 2e-3                   aigbacc = 0.012

+bigbacc = 2.8e-3              cigbacc = 2e-3                nigbacc = 1

+aigbinv = 0.014               bigbinv = 4e-3                cigbinv = 4e-3

+eigbinv = 1.1                 nigbinv = 3                   ntox = 1

+toxref = 1.5e-8               pigcd = 1                     poxedge = 1

+dlcig = 0                     aigsd = 0.012                 bigsd = 2.8e-3

+cigsd = 2e-3                  elm = 0                       xpart = 0

+cgso = cgon3iso               cgdo = cgon3iso               cgbo = 1e-13

+ckappa = 0.6                  cf = 0                        clc = 1e-7

+cle = 0.6                     dlc = 8.5e-9                  dwc = 0

+vfbcv = -1                    cgsl = 9e-11                  cgdl = 9e-11

+acde = 0.5                    calcacm = 1                   ldif = 1e-7

+hdif = 4e-7                   rsh = 87                      rd = 0

+rs = 0                        rsc = 0                       rdc = 0

+cj = cjn3iso                  mj = 0.269967                 mjsw = 8.18799e-2

+cjsw = cjswn3iso              cjswg = cjswgn3iso            mjswg = 0.185718

+js = 2.35e-5                  jsw = 1e-11                   pbsw = 0.112146

+pb = 0.632866                 pbswg = 0.137237              tnom = 25

+ute = -1.5                    lute = -3.3e-7                wute = 0

+pute = 2.8e-13                kt1 = -0.372                  lkt1 = -1.323489e-23

+wkt1 = 1.58e-8                pkt1 = 4e-14                  kt1l = 0

+kt2 = -0.0264                 lkt2 = 1.012e-8               wkt2 = 7.92e-9

+pkt2 = 2.16e-15               ua1 = 3.8635e-9               lua1 = -8.79525e-16

+wua1 = -7e-16                 pua1 = 8.6e-22                ub1 = -6.127e-18

+lub1 = 4.2381e-24             wub1 = 6.1e-25                pub1 = 1.1905e-31

+uc1 = -1.04e-11               luc1 = 6.3336e-17             wuc1 = -9.204e-17

+puc1 = 9.46e-23               at = 8.95e3                   prt = -2.5e3

+pprt = 3.07e-10               xti = 3                       saref = 5e-6

+sbref = 5e-6                  wlod = 0                      ku0 = 0

+kvsat = 0                     kvth0 = 0                     tku0 = 0

+llodku0 = 0                   wlodku0 = 0                   llodvth = 0

+wlodvth = 0                   lku0 = 0                      wku0 = 0

+pku0 = 0                      lkvth0 = 0                    wkvth0 = 0

+pkvth0 = 0                    stk2 = 0                      lodk2 = 1

+steta0 = 0                    lodeta0 = 1                   agidl = 0

+bgidl = 2.3e9                 cgidl = 0.5                   egidl = 0.8

+type = n

//

//

//

//  *************************************************************

//  *                                                           *

//  *               3.3V ISOLATED PMOS DEVICES MODEL                     *

//  *                                                           *

//  *************************************************************

model p3iso bsim3v3

+version = 3.2                 binunit = 2                   mobmod = 1

+capmod = 3                    nqsmod = 0                    igcmod = 0

+igbmod = 0                    lmin = 3.5e-7                 lmax = 2.001e-4

+wmin = 4e-7                   wmax = 2.001e-4                tox = 9.4e-9+dtoxp3iso

+xj = 1e-7                     nch = 1e17                    wint = 1.31559e-7

+wl = -4.77e-14                wln = 1                       ww = -5.53395e-14

+wwn = 1                       wwl = 1.893e-20               lint = 7.6e-8

+ll = -2.57e-14                lln = 1                       lw = -7.1e-15

+lwn = 1                       lwl = 2.87e-21                dwg = 0

+dwb = 0                       xl = -2e-9+dxlp3iso           xw = 3e-9+dxwp3iso

+vth0 = -0.7+dvtp3iso          lvth0 = -2.491e-8             wvth0 = -2e-8

+pvth0 = 1.8e-14               k1 = 0.7155                   lk1 = 8.2875e-8

+wk1 = -8.212727e-8            pk1 = 5e-16                   k2 = -0.01574

+lk2 = -3.639279e-8            wk2 = 1.569708e-8             pk2 = 3.1e-15

+k3 = 0                        k3b = 0                       w0 = 2.5e-6

+nlx = 1.74e-7                 dvt0 = 2.2                    dvt1 = 0.53

+dvt2 = -0.032                 dvt0w = 0                     dvt1w = 5.3e6

+dvt2w = -0.032                ngate = 1e23                  u0 = 0.0165

+lu0 = 0                       wu0 = -1e-9                   pu0 = -3e-16

+ua = 1.7e-9                   lua = -2e-16                  wua = -4e-16

+pua = -1.17e-22               ub = -6.9505e-19              lub = 1.92e-25

+wub = -1.742575e-25           pub = 2e-32                   uc = -5.25e-11

+luc = -1.7925e-17             wuc = -3.222878e-17           puc = -3e-24

+vsat = 9.64e4*dvsatp3iso      wvsat = 3e-3                  pvsat = 0

+a0 = 1.09                     la0 = -3.53e-7                wa0 = 2.482563e-7

+pa0 = 6.3e-14                 ags = 0.15525                 lags = 1.8075e-8

+wags = -1.215e-8              pags = -1.67125e-14           b0 = 0

+b1 = 0                        keta = -8e-3                  lketa = 6.15e-9

+wketa = 1.836e-8              pketa = -4.2e-15              a1 = 0

+a2 = 1                        rdsw = 1.6e3                  prdsw = 0

+prwb = 0                      prwg = 0                      wr = 1

+voff = -0.136                 lvoff = -3e-9                 wvoff = 0

+nfactor = 1.35                lnfactor = 0                  eta0 = 0.08

+etab = -0.07                  dsub = 0.56                   cit = 0

+cdsc = 2.4e-4                 cdscb = 0                     cdscd = 0

+pclm = 0.455                  pdiblc1 = 0.39                pdiblc2 = 7.31e-3

+pdiblcb = 1.5e-4              drout = 2.744                 pscbe1 = 1.7596e9

+pscbe2 = 1.2e-5               pvag = 0                      delta = 0.01

+alpha0 = 1.45e-8              alpha1 = 0.029                beta0 = 19.3

+nigc = 1                      aigc = 0.012                  bigc = 2.8e-3

+cigc = 2e-3                   aigbacc = 0.012               bigbacc = 2.8e-3

+cigbacc = 2e-3                nigbacc = 1                   aigbinv = 0.014

+bigbinv = 4e-3                cigbinv = 4e-3                eigbinv = 1.1

+nigbinv = 3                   ntox = 1                      toxref = 1.5e-8

+pigcd = 1                     poxedge = 1                   dlcig = 0

+aigsd = 0.012                 bigsd = 2.8e-3                cigsd = 2e-3

+elm = 0                       xpart = 0                     cgso = cgop3iso

+cgdo = cgop3iso               cgbo = 1e-13                  ckappa = 0.6

+cf = 0                        clc = 1e-7                    cle = 0.6

+dlc = 0                       dwc = 0                       vfbcv = -1

+cgsl = 1e-10                  cgdl = 1e-10                  acde = 1

+calcacm = 1                   ldif = 1e-7                   hdif = 4e-7

+rsh = 182                     rd = 0                        rs = 0

+rsc = 0                       rdc = 0                       cj = cjp3iso

+mj = 0.433828                 mjsw = 0.244471               cjsw = cjswp3iso

+cjswg = cjswgp3iso            mjswg = 0.249471              js = 1e-4

+jsw = 0                       pbsw = 0.714128               pb = 0.908111

+pbswg = 0.645099              tnom = 25                     ute = -0.725

+pute = 6e-14                  kt1 = -0.528                  pkt1 = 0

+kt1l = 0                      pkt1l = 0                     kt2 = -6.938894e-18

+pkt2 = -6e-15                 ua1 = 4.2565e-9               pua1 = -1.4e-22

+ub1 = -5.2075e-18             pub1 = 5e-32                  uc1 = -2.492e-10

+luc1 = 1.8e-17                puc1 = 0                      at = 3.3e4

+pat = 0                       prt = 6                       xti = 3

+saref = 5e-6                  sbref = 5e-6                  wlod = 0

+ku0 = 0                       kvsat = 0                     kvth0 = 0

+tku0 = 0                      llodku0 = 0                   wlodku0 = 0

+llodvth = 0                   wlodvth = 0                   lku0 = 0

+wku0 = 0                      pku0 = 0                      lkvth0 = 0

+wkvth0 = 0                    pkvth0 = 0                    stk2 = 0

+lodk2 = 1                     steta0 = 0                    lodeta0 = 1

+agidl = 0                     bgidl = 2.3e9                 cgidl = 0.5

+egidl = 0.8                   type = p

//

//  *************************************************************

//  *                                                           *

//  *                3.3V Low Vt NMOS DEVICES MODEL             *

//  *                                                           *

//  *************************************************************

model nl bsim3v3

+version = 3.2                 binunit = 2                   mobmod = 1

+capmod = 3                    nqsmod = 0                    igcmod = 0

+igbmod = 0                    lmin = 3.5e-7                 lmax = 200.001e-6

+wmin = 4e-7                   wmax = 200.001e-6             tox = 8.566e-9+dtoxn3

+xj = 1.5e-7                   nch = 1e17                    wint = 9.3e-8

+wl = -3.806e-14               wln = 1                       ww = -3.887e-14

+wwn = 1                       wwl = 1.375e-20               lint = 0

+ll = 0                        lln = 1                       lw = 1.56e-15

+lwn = 1                       lwl = 5e-23                   dwg = 0

+dwb = 0                       xl = 6e-9+dxln3               xw = -8e-9+dxwn3

+vth0 = 0.515+dvtn3            lvth0 = 3.02e-8               wvth0 = -2.4e-8

+pvth0 = 4.6e-15               k1 = 0.53                     pk1 = 6e-15

+k2 = 0.02418                  lk2 = 0                       wk2 = 2e-9

+pk2 = -3e-15                  k3 = 0                        k3b = 0

+w0 = 2.5e-6                   nlx = 1.74e-7                 dvt0 = 2.2

+dvt1 = 0.53                   dvt2 = -0.032                 dvt0w = 0

+dvt1w = 5.3e6                 dvt2w = -0.032                ngate = 1e23

+u0 = 0.0546*du0n3             lu0 = -1e-9                   wu0 = -8.84e-9

+pu0 = 4.5e-15                 ua = 1.43e-9                  wua = 4e-17

+ub = -2.461e-19               lub = 6e-26                   wub = 0

+pub = 3e-32                   uc = -4.65e-11                luc = 1.3e-17

+wuc = 1e-17                   vsat = 7.459e4                lvsat = 1.715e-3

+pvsat = 6.625e-9              a0 = 0.9                      la0 = -1.75e-7

+wa0 = 2.5125e-7               pa0 = 0                       ags = 0.1

+lags = 1.7e-7                 wags = 2.5e-8                 pags = 0

+b0 = 0                        b1 = 0                        keta = -0.0125

+lketa = -1.701e-8             wketa = 5.3625e-9             pketa = -3.15e-15

+a1 = 0                        a2 = 1                        rdsw = 740

+prdsw = -7e-11                prwb = 0                      prwg = 0

+wr = 1                        voff = -0.08                  nfactor = 0.885

+lnfactor = -1e-7              pnfactor = 0                  eta0 = 0.08

+weta0 = 0                     etab = 0                      letab = 0

+wetab = 0                     dsub = 0.56                   cit = 0

+cdsc = 2.4e-4                 cdscb = 0                     cdscd = 0

+pclm = 1.3                    pdiblc1 = 0.39                ppdiblc1 = -5.265e-13

+pdiblc2 = 8.6e-3              pdiblcb = 0                   drout = 1.68

+pscbe1 = 4.24e8               pscbe2 = 1e-5                 pvag = 0

+delta = 0                     alpha0 = 6.28e-7              alpha1 = 5.75

+beta0 = 20.2                  nigc = 1                      aigc = 0.012

+bigc = 2.8e-3                 cigc = 2e-3                   aigbacc = 0.012

+bigbacc = 2.8e-3              cigbacc = 2e-3                nigbacc = 1

+aigbinv = 0.014               bigbinv = 4e-3                cigbinv = 4e-3

+eigbinv = 1.1                 nigbinv = 3                   ntox = 1

+toxref = 1.5e-8               pigcd = 1                     poxedge = 1

+dlcig = 0                     aigsd = 0.012                 bigsd = 2.8e-3

+cigsd = 2e-3                  elm = 0                       xpart = 0

+cgso = cgon3                  cgdo = cgon3                  cgbo = 5.0022e-14

+ckappa = 0.6                  cf = 0                        clc = 1e-7

+cle = 0.6                     dlc = 8.5e-9                  dwc = 0

+vfbcv = -1                    cgsl = 9e-11                  cgdl = 9e-11

+calcacm = 1                   ldif = 1e-7                   hdif = 4e-7

+rsh = 95                      rd = 0                        rs = 0

+rsc = 0                       rdc = 0                       cj = cjn3

+mj = 0.269967                 mjsw = 8.18799e-2             cjsw = cjswn3

+cjswg = cjswgn3               mjswg = 0.185718              js = 1.52025e-5

+jsw = 6e-12                   pbsw = 0.112146               pb = 0.632866

+pbswg = 0.137237              tnom = 25                     ute = -1.5

+lute = 1e-7                   wute = 4e-8                   pute = 0

+kt1 = -0.33275                lkt1 = -3.575e-8              wkt1 = 0

+pkt1 = 2e-14                  kt1l = 0                      kt2 = -0.033

+lkt2 = 0                      wkt2 = 0                      pkt2 = 0

+ua1 = 4.0945e-9               lua1 = -1e-16                 wua1 = -1.4465e-15

+pua1 = 3e-22                  ub1 = -7.2295e-18             lub1 = -1e-25

+wub1 = 2.413e-24              pub1 = -4e-31                 uc1 = -5.6e-11

+luc1 = 0                      wuc1 = 0                      puc1 = 0

+at = 3.3e4                    prt = 110                     xti = 3

+tpb = 1.716697e-3             tpbsw = 7.957164e-4           tpbswg = 6.83e-4

+tcj = 9.707176e-4             tcjsw = 1.321796e-3           tcjswg = 1.72e-3

+saref = 5e-6                  sbref = 5e-6                  wlod = 0

+ku0 = 0                       kvsat = 0                     kvth0 = 0

+tku0 = 0                      llodku0 = 0                   wlodku0 = 0

+llodvth = 0                   wlodvth = 0                   lku0 = 0

+wku0 = 0                      pku0 = 0                      lkvth0 = 0

+wkvth0 = 0                    pkvth0 = 0                    stk2 = 0

+lodk2 = 1                     steta0 = 0                    lodeta0 = 1

+agidl = 0                     bgidl = 2.3e9                 cgidl = 0.5

+egidl = 0.8                   type = n

//

//  *************************************************************

//  *                                                           *

//  *                3.3V Low Vt PMOS DEVICES MODEL             *

//  *                                                           *

//  *************************************************************

model pl bsim3v3

+version = 3.2                 binunit = 2                   mobmod = 1

+capmod = 3                    nqsmod = 0                    igcmod = 0

+igbmod = 0                    lmin = 3.5e-7                 lmax = 200.001e-6

+wmin = 4e-7                   wmax = 200.001e-6                   tox = 7.1e-9+dtoxp3

+xj = 1.5e-7                   nch = 1e17                    wint = 1.81e-7

+wl = -6.986e-14               wln = 1                       ww = -7.987e-14

+wwn = 1                       wwl = 2.9025e-20              lint = 0

+ll = 0                        lln = 1                       lw = 2.208e-14

+lwn = 1                       lwl = -6.55375e-21            dwg = 0

+dwb = 0                       xl = -3e-9+dxlp3              xw = 1e-8+dxwp3

+vth0 = -0.54+dvtp3            lvth0 = -2.795e-8             wvth0 = 2.6e-9

+pvth0 = 2.1e-14               k1 = 0.693625                 lk1 = 0

+wk1 = 3.923e-8                pk1 = -5e-15                  k2 = -0.0186

+lk2 = 1.1e-8                  wk2 = 5.897363e-9             pk2 = 2.1e-16

+k3 = 0                        k3b = 0                       w0 = 2.5e-6

+nlx = 1.74e-7                 dvt0 = 2.2                    dvt1 = 0.53

+dvt2 = -0.032                 dvt0w = 0                     dvt1w = 5.3e6

+dvt2w = -0.032                ngate = 1e23                  u0 = 0.015

+lu0 = -2.7825e-9              wu0 = -3.2345e-9              pu0 = 8.51e-16

+ua = 1.0125e-9                lua = -3.99e-16               wua = -5.208e-16

+pua = 1.26e-22                ub = 5.6895e-19               lub = 2.415e-25

+wub = -5.37e-26               pub = 4.11e-32                uc = -4.65e-11

+luc = -8.71875e-18            wuc = 1.8e-18                 puc = 1.5605e-23

+vsat = 5.07e4*dvsatp3         lvsat = -1.22e-3              pvsat = 0

+a0 = 1                        la0 = -3e-7                   wa0 = 0

+pa0 = 0                       ags = 0.17                    lags = 1.9e-7

+wags = -5e-8                  pags = 0                      b0 = 0

+b1 = 0                        keta = -4e-4                  lketa = -8.69e-9

+wketa = 1.5e-9                pketa = -5.38e-16             a1 = 0

+a2 = 1                        rdsw = 1.6e3                  prdsw = -3.3e-10

+prwb = 0                      prwg = 0                      wr = 1

+voff = -0.18                  lvoff = 0                     pvoff = 0

+nfactor = 2.97                lnfactor = 4e-7               pnfactor = 0

+eta0 = 0.08                   etab = -0.014                 dsub = 0.56

+cit = 0                       cdsc = 2.4e-4                 cdscb = 0

+cdscd = 0                     pclm = 2.145                  ppclm = 0

+pdiblc1 = 2.775558e-17        pdiblc2 = 8.6e-3              pdiblcb = 0

+drout = 0.56                  pscbe1 = 7.208e8              pscbe2 = 1e-5

+pvag = 0                      delta = 0.01                  alpha0 = 8.87845e-9

+alpha1 = 0.065                beta0 = 19.88                 nigc = 1

+aigc = 0.012                  bigc = 2.8e-3                 cigc = 2e-3

+aigbacc = 0.012               bigbacc = 2.8e-3              cigbacc = 2e-3

+nigbacc = 1                   aigbinv = 0.014               bigbinv = 4e-3

+cigbinv = 4e-3                eigbinv = 1.1                 nigbinv = 3

+ntox = 1                      toxref = 1.5e-8               pigcd = 1

+poxedge = 1                   dlcig = 0                     aigsd = 0.012

+bigsd = 2.8e-3                cigsd = 2e-3                  elm = 0

+xpart = 0                     cgso = cgop3                  cgdo = cgop3

+cgbo = 0                      ckappa = 0.6                  cf = 0

+clc = 0                       cle = 1                       dlc = 0

+dwc = 0                       vfbcv = -1                    noff = 1

+acde = 1                      moin = 15                     cgsl = 3e-11

+cgdl = 3e-11                  calcacm = 1                   ldif = 1.2e-7

+hdif = 4.05e-7                rsh = 182                     rd = 0

+rs = 0                        rsc = 0                       rdc = 0

+cj = cjp3                     mj = 0.433828                 mjsw = 0.244471

+cjsw = cjswp3                 cjswg = cjswgp3               mjswg = 0.249471

+js = 1.97e-4                  jsw = 7e-11                   pbsw = 0.714128

+pb = 0.908111                 pbswg = 0.645099              tnom = 25

+ute = -0.77625                lute = 1.5e-7                 wute = -1.0125e-7

+pute = -1.85e-14              kt1 = -0.57475                lkt1 = -2e-8

+wkt1 = -6.2325e-8             pkt1 = 0                      kt1l = 0

+kt2 = -0.0517                 wkt2 = -1.551e-8              pkt2 = 0

+ua1 = 3.2325e-9               lua1 = -2e-16                 wua1 = -1.004875e-15

+pua1 = 7e-23                  ub1 = -3.3245e-18             lub1 = 2.871e-25

+wub1 = 1.07735e-24            pub1 = -3.44e-31              uc1 = -5.6e-11

+wuc1 = 0                      puc1 = -1.68e-23              at = 9.9e3

+pat = 4.1e-9                  prt = 500                     pprt = 2.9e-10

+xti = 3                       tpb = 1.590593e-3             tpbsw = 1.585214e-3

+tcj = 9.147986e-4             tcjsw = 6.63089e-4            saref = 5e-6

+sbref = 5e-6                  wlod = 0                      ku0 = 0

+kvsat = 0                     kvth0 = 0                     tku0 = 0

+llodku0 = 0                   wlodku0 = 0                   llodvth = 0

+wlodvth = 0                   lku0 = 0                      wku0 = 0

+pku0 = 0                      lkvth0 = 0                    wkvth0 = 0

+pkvth0 = 0                    stk2 = 0                      lodk2 = 1

+steta0 = 0                    lodeta0 = 1                   agidl = 0

+bgidl = 2.3e9                 cgidl = 0.5                   egidl = 0.8

+type = p

//

endsection mos

//  ***********************************************************************

//  ******************************************************************************

//  *                                                                            *

//  *                           5.0V MOS DEVICES MODELS                          *

//  *                                                                            *

//  ******************************************************************************

//

section mos_hv

//  *************************************************************

//  *                                                           *

//  *               5.0V NMOS DEVICES MODEL                     *

//  *                                                           *

//  *************************************************************

model n5 bsim3v3 type=n

+ lmin=5.0e-7               lmax=2.001e-4               wmin=4.0e-7

+ wmax=2.001e-4               tnom=25.0                 version=3.2

+ tox=toxnl                 toxm=toxnl                xj=1.0000000e-07

+ nch=9.6300000e+16         lln=2.0000000             lwn=0.9500000

+ wln=0.3971576             wwn=1.9979510             lint=2.1000000e-08

+ ll=-7.5807710e-21         lw=0.00                   lwl=1.1003430e-26

+ wint=1.0000000e-07        wl=-2.8550501e-10         ww=-8.4100130e-21

+ wwl=0.00                  mobmod=1                  binunit=2

+ xl=0.00 + dxlnl           xw=0.00 + dxwnl           lmlt=1

+ wmlt=1                    binflag=0                 dwg=-3.4400000e-09

+ dwb=3.8600000e-09         ldif=1.00e-07             hdif=4.50e-07

+ rsh=85.98                 rd=0                      rs=0

+ rsc=0                     rdc=0                     vth0=0.6483840 + dvtnl

+ lvth0=2.5e-08             wvth0=-3.7800000e-08      pvth0=1.2414400e-14

+ k1=1.0182000              k2=-9.7500000e-03         lk2=4.2000000e-09

+ pk2=-1.1500000e-15        k3=-5.0500000e-02         dvt0=29.8447020

+ dvt1=1.16892              dvt2=-3.5625000e-02       dvt0w=-4.1530000

+ dvt1w=2.0600000e+06       dvt2w=-0.2970000          nlx=2.6199999e-07

+ w0=1.2900000e-07          k3b=1.8200001e-02         ngate=1.0000000e+30

+ vfb=-1.0875350            vsat=2.4900000e+05 * dvsatnl

+ lvsat=5.7000000e-02       pvsat=-2.2335000e-08      ua=7.0000000e-10

+ ub=9.8572000e-19          uc=3.8805000e-11          rdsw=1.2780000e+03

+ prdsw=-7.5050000e-11      prwb=8.2200000e-02        prwg=2.1000000e-09

+ wr=1.0630000              u0=5.2500000e-02          lu0=1.2528000e-08

+ wu0=-3.6200000e-09        pu0=-6.0984000e-15        a0=0.9991000

+ keta=0.00                 lketa=-3.9360000e-09      wketa=8.1000000e-09

+ pketa=1.1740000e-14       a1=0.00                   a2=0.1640000

+ ags=0.1300000             b0=9.9900000e-06          b1=4.1400000e-05

+ voff=-0.1078000           nfactor=1.4470400         cit=-2.8899999e-04

+ cdsc=7.0500000e-03        cdscb=-2.3500000e-03      cdscd=1.8500000e-03

+ eta0=4.5000000e-02        etab=-6.0000010e-02       dsub=0.6160000

+ pclm=2.5171001            pdiblc1=3.2500000e-02     pdiblc2=1.0000000e-05

+ pdiblcb=-0.1000000        drout=0.4003800           pscbe1=4.5100000e+08

+ pscbe2=7.5000000e-07      pvag=0.00                 delta=5.2500000e-02

+ alpha0=0.00               alpha1=0.00               beta0=30.0000000

+ kt1=-0.4634499            kt2=-6.9851240e-02        at=1.7352000e+05

+ pat=-9.4400000e-09        ute=-1.9949999            ua1=-3.1600000e-11

+ wua1=-3.3915000e-16       ub1=-5.9755000e-19        uc1=-7.1200000e-13

+ kt1l=-2.0520000e-08       prt=14.9999980            cj=cjnl

+ mj=0.321                  pb=0.681                  cjsw=cjswnl

+ mjsw=0.265                pbsw=0.569                cjswg=cjswgnl

+ mjswg=1.36                pbswg=1.56                tpb=0.00174

+ tpbsw=0.00175             tcj=0.000999              tcjsw=0.00109

+ js=6.50e-04               jsw=6.00e-11              nj=1.0

+ xti=3.0                   cgdo=cgonl                cgso=cgonl

+ cgbo=1.0e-13              capmod=3                  nqsmod=0

+ elm=5                     xpart=0                   cgsl=7.0000000e-11

+ cgdl=7.0000000e-11        ckappa=0.6000000          cf=0.00

+ clc=1.0000000e-07         cle=0.6000000             dlc=3.5e-08

+ dwc=0                     vfbcv=-0.764247           llc=0

+ lwc=0                     wlc=0                     wwc=0

+ lwlc=0                    wwlc=0                    acde=0.5

+ moin=15                   noff=1                    voffcv=0

//

//  *************************************************************

//  *                                                           *

//  *               5.0V PMOS DEVICES MODEL                     *

//  *                                                           *

//  *************************************************************

model p5 bsim3v3 type=p

+ lmin=5.0e-7               lmax=2.001e-4               wmin=4.0e-7

+ wmax=2.001e-4               tnom=25.0                 version=3.2

+ tox=toxpl                 toxm=toxpl                xj=1.0000000e-07

+ nch=4.0373000e+16         lln=1.9100790             lwn=1.0000000

+ wln=0.8209912             wwn=1.3720000             lint=-7.0000000e-10

+ ll=-1.7837319e-20         lw=0.00                   lwl=1.2889520e-26

+ wint=9.0000000e-08        wl=-2.8751951e-14         ww=-1.8572960e-16

+ wwl=-7.6600000e-23        mobmod=1                  binunit=2

+ xl=0.00 + dxlpl           xw=0.00 + dxwpl           lmlt=1

+ wmlt=1                    binflag=0                 dwg=-1.8080900e-08

+ dwb=1.0847200e-09         ldif=1.00e-07             hdif=4.50e-07

+ rsh=178.6                 rd=0                      rs=0

+ rsc=0                     rdc=0

+ vth0= - 1.0000000 + dvtpl + 0.05                    lvth0=-4.0e-08

+ pvth0=-1.2500000e-14      k1=0.6105757              k2=9.5909850e-02

+ pk2=1.1000000e-14         k3=0.3675160              dvt0=3.6057210

+ dvt1=0.842726             dvt2=-4.5695090e-03       dvt0w=-2.6849581e-02

+ dvt1w=0.00                dvt2w=0.3798480           nlx=1.3670000e-07

+ w0=-1.7635500e-07         k3b=2.0408060e-02         ngate=1.0000000e+30

+ vfb=-0.3059404            vsat=1.2848e+05 * dvsatpl

+ pvsat=-5.1075000e-09      ua=3.1000000e-09          ub=1.0400003e-20

+ uc=-5.0306100e-11         rdsw=1.2415624e+03        prdsw=1.5276000e-10

+ prwb=-0.2000000           prwg=-5.5161140e-03       wr=1.2210090

+ u0=2.220e-02              wu0=-3.4132380e-09        a0=1.4720327

+ pa0=4.2000000e-13         keta=1.7888000e-02        lketa=-6.6000000e-09

+ wketa=1.7973100e-09       pketa=9.7786000e-15       a1=0.00

+ a2=0.4000000              ags=0.1055540             lags=7.8700010e-09

+ pags=-7.5000000e-15       b0=1.5007200e-07          b1=1.4900000e-07

+ voff=-0.2140697           nfactor=2.0000000         cit=-3.2557340e-04

+ cdsc=1.5354051e-03        cdscb=0.00                cdscd=3.0550180e-04

+ eta0=1.1696520            etab=-0.2000000           dsub=1.5253700

+ pclm=3.9440000            pdiblc1=0.1000000         pdiblc2=2.9032939e-04

+ pdiblcb=-0.2000000        drout=0.4670055           pscbe1=1.7535000e+09

+ pscbe2=1.2732499e-06      pvag=1.0000000            delta=1.0000000e-02

+ alpha0=0.00               alpha1=0.00               beta0=30.0000000

+ kt1=-0.5311401            kt2=-5.5397390e-02        at=1.5319998e+02

+ ute=-1.4573530            pute=8.0000000e-14        ua1=9.7720690e-11

+ lua1=-4.6560000e-16       wua1=3.3003000e-16        pua1=3.4500000e-22

+ ub1=-2.4118370e-18        uc1=-6.3786870e-11        kt1l=-6.4413500e-08

+ prt=9.9999900             cj=cjpl                   mj=0.4400759

+ pb=0.8970423              cjsw=cjswpl               mjsw=0.1815828

+ pbsw=0.6241563            cjswg=cjswgpl             mjswg=0.2600447

+ pbswg=0.59718             tpb=1.656768e-03          tpbsw=1.77291e-03

+ tcj=9.976401e-04          tcjsw=6.58287e-04         js=2.50e-04

+ jsw=2.50e-11              nj=1.0                    xti=3.0

+ cgdo=cgopl                cgso=cgopl                cgbo=1.0e-13

+ capmod=3                  nqsmod=0                  elm=5

+ xpart=0                   cgsl=4.9850000e-11        cgdl=4.9850000e-11

+ ckappa=0.6000000          cf=0.00                   clc=1.0000000e-07

+ cle=0.6000000             dlc=3.8e-08               dwc=0

+ vfbcv=-0.213              llc=0                     lwc=0

+ wlc=0                     wwc=0                     lwlc=0

+ wwlc=0                    acde=0.5                  moin=15

+ noff=1                    voffcv=0

//  *************************************************************

//  *                                                           *

//  *               5.0V ISOLATED NMOS DEVICE MODEL            *

//  *                                                           *

//  *************************************************************

model n5iso bsim3v3

+version = 3.2                 binunit = 2                   mobmod = 1

+capmod = 3                    nqsmod = 0                    igcmod = 0

+igbmod = 0                    lmin = 5.5e-7                 lmax = 2.001e-4

+wmin = 4e-7                   wmax = 2.001e-4                   tox = 1.49e-8+dtoxnliso

+xj = 1e-7                     nch = 1e17                    wint = 1.1e-7

+wl = -1.5e-13                 wln = 1                       ww = -4.5e-14

+wwn = 1                       wwl = 6e-20                   lint = 0

+ll = 0                        lln = 1                       lw = 2e-14

+lwn = 1                       lwl = -8e-21                  dwg = 0

+dwb = 0                       xl = 0+dxlnliso               xw = 3e-9+dxwnliso

+vth0 = 0.68+dvtnliso          lvth0 = 1.1723e-7             wvth0 = -5e-8

+pvth0 = -2.7e-14              k1 = 1.007                    lk1 = 0

+wk1 = 0                       pk1 = -1e-13                  k2 = -0.01023

+lk2 = -4.3502e-8              wk2 = -5.381425e-8            pk2 = 3.269e-14

+k3 = 0.25                     k3b = -19.825                 w0 = 2.5e-6

+nlx = 1.74e-7                 dvt0 = 2.2                    dvt1 = 0.53

+dvt2 = -0.032                 dvt0w = 0                     dvt1w = 5.3e6

+dvt2w = -0.032                ngate = 1e23                  u0 = 0.059

+lu0 = 1.935e-8                wu0 = -6.05e-9                pu0 = -5e-15

+ua = 1.035e-9                 lua = 5.2325e-16              wua = -2.25e-16

+pua = -2.28e-22               ub = 1.08008e-18              lub = -1.44925e-24

+wub = 1.1e-25                 pub = -9.82545e-31            uc = 5.8125e-11

+luc = -6.898275e-17           wuc = -3.72e-17               puc = -7.32375e-23

+vsat = 6.767e4*dvsatnliso     pvsat = 2.7e-9                a0 = 0.95

+la0 = -8.24e-7                wa0 = 1e-8                    pa0 = 2.2675e-13

+ags = 0.125                   lags = -1.22e-7               wags = -1.875e-8

+pags = 1.294375e-13           b0 = 0                        b1 = 0

+keta = 0.0115                 lketa = -3.606e-8             wketa = -3.105e-9

+pketa = 2.1015e-14            a1 = 0                        a2 = 1

+rdsw = 1.755e3                prdsw = 0                     prwb = 0

+prwg = 0                      wr = 1                        voff = -0.132

+lvoff = -3e-8                 pvoff = 1e-14                 nfactor = 1

+eta0 = 0.08                   etab = -0.07                  dsub = 0.56

+cit = 0                       cdsc = 2.4e-4                 cdscb = 0

+cdscd = 0                     pclm = 1.3                    pdiblc1 = 0.39

+pdiblc2 = 8.6e-3              pdiblcb = 0                   drout = 0.98

+pscbe1 = 5.512e8              pscbe2 = 1e-5                 pvag = 0

+delta = 0.01                  alpha0 = 4.7e-7               alpha1 = 5.4

+beta0 = 30                    nigc = 1                      aigc = 0.012

+bigc = 2.8e-3                 cigc = 2e-3                   aigbacc = 0.012

+bigbacc = 2.8e-3              cigbacc = 2e-3                nigbacc = 1

+aigbinv = 0.014               bigbinv = 4e-3                cigbinv = 4e-3

+eigbinv = 1.1                 nigbinv = 3                   ntox = 1

+toxref = 1.5e-8               pigcd = 1                     poxedge = 1

+dlcig = 0                     aigsd = 0.012                 bigsd = 2.8e-3

+cigsd = 2e-3                  elm = 0                       xpart = 0

+cgso = cgonliso               cgdo = cgonliso               cgbo = 1e-13

+ckappa = 0.6                  cf = 0                        clc = 1e-7

+cle = 0.6                     dlc = 3.5e-8                  dwc = 0

+vfbcv = -1                    cgsl = 7e-11                  cgdl = 7e-11

+acde = 0.5                    calcacm = 1                   ldif = 1e-7

+hdif = 4e-7                   rsh = 87                      rd = 0

+rs = 0                        rsc = 0                       rdc = 0

+cj = cjnliso                  mj = 0.321                    mjsw = 0.265

+cjsw = cjswnliso              cjswg = cjswgnliso            mjswg = 1.36

+js = 1e-6                     jsw = 1e-12                   pbsw = 0.569

+pb = 0.681                    pbswg = 1.56                  tnom = 25

+ute = -1.5                    lute = -6.83e-7               wute = 1e-7

+pute = 2.33975e-13            kt1 = -0.3835                 lkt1 = -6e-8

+wkt1 = -4.4e-8                pkt1 = 4.014375e-14           kt1l = 0

+kt2 = -0.08278                lkt2 = 4.18e-8                wkt2 = 2.09e-8

+pkt2 = -2.3595e-14            ua1 = 3.5635e-9               lua1 = -3.865071e-15

+wua1 = 0                      pua1 = 6.5463e-22             ub1 = -6.088e-18

+lub1 = 1.098492e-23           wub1 = 0                      pub1 = -2.4296e-30

+uc1 = -2.24e-11               luc1 = 2.82135e-16            wuc1 = 0

+puc1 = -7.526471e-23          at = 1.98e4                   prt = -1.47e3

+wprt = 0                      pprt = 6.239425e-10           xti = 3

+saref = 5e-6                  sbref = 5e-6                  wlod = 0

+ku0 = 0                       kvsat = 0                     kvth0 = 0

+tku0 = 0                      llodku0 = 0                   wlodku0 = 0

+llodvth = 0                   wlodvth = 0                   lku0 = 0

+wku0 = 0                      pku0 = 0                      lkvth0 = 0

+wkvth0 = 0                    pkvth0 = 0                    stk2 = 0

+lodk2 = 1                     steta0 = 0                    lodeta0 = 1

+agidl = 0                     bgidl = 2.3e9                 cgidl = 0.5

+egidl = 0.8                   type = n                          //

//  *************************************************************

//  *                                                           *

//  *               5.0V ISOLATED PMOS DEVICES MODEL           *

//  *                                                           *

//  *************************************************************

model p5iso bsim3v3

+version = 3.2                 binunit = 2                   mobmod = 1

+capmod = 3                    nqsmod = 0                    igcmod = 0

+igbmod = 0                    lmin = 5e-7                   lmax = 2.001e-4

+wmin = 4e-7                   wmax = 2.001e-4                   tox = 1.54e-8+dtoxpliso

+xj = 1e-7                     nch = 1e17                    wint = 1.35e-7

+wl = -1.93e-14                wln = 1                       ww = -5.5e-14

+wwn = 1                       wwl = 6.5e-21                 lint = 0

+ll = 0                        lln = 1                       lw = 0

+lwn = 1                       lwl = 0                       dwg = 0

+dwb = 0                       xl = 0+dxlpliso               xw = 3e-9+dxwpliso

+vth0 = -1.11+dvtpliso         lvth0 = 1.54e-8               wvth0 = -2e-8

+pvth0 = 3e-14                 k1 = 0.9045                   lk1 = 2e-8

+wk1 = -2e-8                   pk1 = -3e-14                  k2 = -9.3e-3

+lk2 = -3.29e-9                wk2 = 2.0615e-8               pk2 = 6.08e-15

+k3 = 0                        k3b = 0                       w0 = 2.5e-6

+nlx = 1.74e-7                 dvt0 = 2.2                    dvt1 = 0.53

+dvt2 = -0.032                 dvt0w = 0                     dvt1w = 5.3e6

+dvt2w = -0.032                ngate = 1e23                  u0 = 0.02

+lu0 = -8e-10                  wu0 = -2.3e-9                 pu0 = 3.944305e-31

+ua = 2.05e-9                  lua = -4.5e-16                wua = -5e-16

+pua = -9e-23                  ub = 6.37e-19                 lub = 1.33205e-24

+wub = -3.1e-25                pub = -7e-32                  uc = -3.8525e-11

+luc = 9.2575e-17              wuc = -5.023e-17              puc = -1.8e-23

+vsat = 7.919e4*dvsatpliso     a0 = 1.1                      la0 = -5.52e-7

+wa0 = 1e-7                    pa0 = 1.11e-13                ags = 0.091

+lags = 1.9125e-7              wags = -7e-9                  pags = -5.9e-14

+b0 = 0                        b1 = 0                        keta = 0.01825

+lketa = -1.92625e-8           wketa = -1.5e-9               pketa = 3.5e-15

+a1 = 0                        a2 = 1                        rdsw = 100

+prwb = 0                      prwg = 0                      wr = 1

+voff = -0.156                 lvoff = -1e-8                 wvoff = -3.308722e-24

+pvoff = 0                     nfactor = 0.85                lnfactor = 1.275e-7

+wnfactor = 5.25e-8            pnfactor = -2e-13             eta0 = 0.08

+etab = -0.07                  dsub = 0.56                   cit = 0

+cdsc = 2.4e-4                 cdscb = 0                     cdscd = 0

+pclm = 1.3                    pdiblc1 = 0.39                pdiblc2 = 8.6e-3

+pdiblcb = 0                   drout = 0.71                  pscbe1 = 1.2932e9

+pscbe2 = 1.8e-5               pvag = 0                      delta = 0.01

+alpha0 = 1.99e-7              alpha1 = 0.32                 beta0 = 26

+nigc = 1                      aigc = 0.012                  bigc = 2.8e-3

+cigc = 2e-3                   aigbacc = 0.012               bigbacc = 2.8e-3

+cigbacc = 2e-3                nigbacc = 1                   aigbinv = 0.014

+bigbinv = 4e-3                cigbinv = 4e-3                eigbinv = 1.1

+nigbinv = 3                   ntox = 1                      toxref = 1.5e-8

+pigcd = 1                     poxedge = 1                   dlcig = 0

+aigsd = 0.012                 bigsd = 2.8e-3                cigsd = 2e-3

+elm = 0                       xpart = 0                     cgso = cgopliso

+cgdo = cgopliso               cgbo = 1e-13                  ckappa = 0.6

+cf = 0                        clc = 1e-7                    cle = 0.6

+dlc = 3.8e-8                  dwc = 0                       vfbcv = -1

+cgsl = 4.985e-11              cgdl = 4.985e-11              calcacm = 1

+ldif = 1e-7                   hdif = 4e-7                   rsh = 182

+rd = 0                        rs = 0                        rsc = 0

+rdc = 0                       cj = cjpliso                  mj = 0.440076

+mjsw = 0.181583               cjsw = cjswpliso              cjswg = cjswgpliso

+mjswg = 0.260045              js = 8e-7                     jsw = 1.1e-12

+pbsw = 0.624156               pb = 0.897042                 pbswg = 0.59718

+tnom = 25                     ute = -0.94                   lute = 0

+wute = -1e-8                  pute = 2.997e-13              kt1 = -0.6

+lkt1 = -4.8e-8                wkt1 = -7e-8                  pkt1 = 9e-14

+kt1l = 0                      wkt1l = 0                     kt2 = -0.1672

+lkt2 = 3.87e-8                wkt2 = -7.68e-8               pkt2 = 6.07e-14

+ua1 = 3.51e-9                 lua1 = 0                      wua1 = 1e-16

+pua1 = 1.00815e-21            ub1 = -5.01e-18               lub1 = -1e-24

+wub1 = -1.7e-25               pub1 = -1.35e-30              uc1 = -5.6e-11

+luc1 = -5.24e-17              wuc1 = 1.986e-16              puc1 = -1.14397e-22

+at = 3.3e4                    prt = 50                      pprt = 4.2e-10

+xti = 3                       saref = 5e-6                  sbref = 5e-6

+wlod = 0                      ku0 = 0                       kvsat = 0

+kvth0 = 0                     tku0 = 0                      llodku0 = 0

+wlodku0 = 0                   llodvth = 0                   wlodvth = 0

+lku0 = 0                      wku0 = 0                      pku0 = 0

+lkvth0 = 0                    wkvth0 = 0                    pkvth0 = 0

+stk2 = 0                      lodk2 = 1                     steta0 = 0

+lodeta0 = 1                   agidl = 0                     bgidl = 2.3e9

+cgidl = 0.5                   egidl = 0.8                      type = p

//  *************************************************************

//  *                                                           *

//  *               5.0V low vt PMOS DEVICES MODEL                     *

//  *                                                           *

//  *************************************************************

model p5l bsim3v3

+version = 3.2                 binunit = 2                   mobmod = 1

+capmod = 3                    nqsmod = 0                    igcmod = 0

+igbmod = 0                    lmin = 0.5E-6                 lmax = 200.001E-6

+wmin = 0.4E-6                 wmax = 200.001E-6              tox = 1.6e-8+dtoxp5l

+xj = 1e-7                     nch = 1e17                    wint = 0

+wl = 0                        wln = 1                       ww = -6e-14

+wwn = 1                       wwl = 1.3e-20                 lint = 0

+ll = -1.5e-14                 lln = 1                       lw = 0

+lwn = 1                       lwl = 1.6e-20                 dwg = 0

+dwb = 0                       xl = 6e-9+dxlp5l              xw = 7e-9+dxwp5l

+vth0 = -0.81+dvtp5l           lvth0 = -5.5e-8               wvth0 = 1e-8

+pvth0 = 2e-14                 k1 = 0.53                     k2 = 0.14787

+lk2 = 0                       wk2 = 3.076e-8                pk2 = -1.188e-14

+k3 = 0                        k3b = 0                       w0 = 2.5e-6

+nlx = 1.74e-7                 dvt0 = 2.2                    dvt1 = 0.53

+dvt2 = -0.032                 dvt0w = 0                     dvt1w = 5.3e6

+dvt2w = -0.032                ngate = 1e23                  u0 = 0.014695

+lu0 = 1.8e-9                  wu0 = -7e-9                   pu0 = -9e-17

+ua = 3e-10                    lua = 4e-17                   wua = -1.065e-15

+pua = 3.475e-22               ub = 1.90775e-18              lub = 8.2635e-25

+wub = 2.7635e-25              pub = -8.2635e-31             uc = -2.79e-11

+luc = 7.6725e-17              wuc = -6.975e-18              puc = -5.58e-23

+vsat = 8e4*dvsatp5l           pvsat = -7.2e-9               a0 = 1.85

+la0 = -9e-7                   wa0 = 3.9e-7                  pa0 = 3e-13

+ags = 0.105                   lags = 7e-8                   wags = -1.7e-8

+pags = -6.310887e-30          b0 = 0                        b1 = 0

+keta = 0.021                  lketa = -1.2e-8               wketa = 8e-9

+pketa = 1.5e-14               a1 = 0                        a2 = 1

+rdsw = 700                    prwb = 0                      prwg = 0

+wr = 1                        voff = -0.164                 nfactor = 1

+eta0 = 0.08                   etab = -0.07                  dsub = 0.56

+cit = 0                       cdsc = 2.4e-4                 cdscb = 0

+cdscd = 0                     pclm = 3.055                  ppclm = -4.8e-13

+pdiblc1 = 0.39                ppdiblc1 = -1.053e-12         pdiblc2 = 8.6e-3

+ppdiblc2 = 0                  pdiblcb = 0                   drout = 1.736

+pdrout = 0                    pscbe1 = 5.512e8              ppscbe1 = 0

+pscbe2 = 1e-5                 pvag = 0                      delta = 0.01

+alpha0 = 2.76e-7              alpha1 = 0.451                beta0 = 28

+nigc = 1                      aigc = 0.012                  bigc = 2.8e-3

+cigc = 2e-3                   aigbacc = 0.012               bigbacc = 2.8e-3

+cigbacc = 2e-3                nigbacc = 1                   aigbinv = 0.014

+bigbinv = 4e-3                cigbinv = 4e-3                eigbinv = 1.1

+nigbinv = 3                   ntox = 1                      toxref = 1.5e-8

+pigcd = 1                     poxedge = 1                   dlcig = 0

+aigsd = 0.012                 bigsd = 2.8e-3                cigsd = 2e-3

+elm = 0                       xpart = 0                     cgso = cgop5l

+cgdo = cgop5l                 cgbo = 1e-13                  ckappa = 0.6

+cf = 0                        clc = 1e-7                    cle = 0.6

+dlc = 3.8e-8                  dwc = 0                       vfbcv = -1

+cgsl = 4.985e-11              cgdl = 4.985e-11              calcacm = 1

+ldif = 1.2e-7                 hdif = 4.5e-7                 rsh = 185

+rd = 0                        rs = 0                        rsc = 0

+rdc = 0                       cj = cjp5l                    mj = 0.440076

+mjsw = 0.181583               cjsw = cjswp5l                cjswg = cjswgp5l

+mjswg = 0.260045              js = 1e-4                     jsw = 0

+pbsw = 0.624156               pb = 0.897042                 pbswg = 0.59718

+tnom = 25                     ute = -0.925                  lute = 0

+wute = 1.45e-7                pute = 0                      kt1 = -0.715

+kt1l = 0                      kt2 = -0.0847                 ua1 = 2.81e-9

+ub1 = -3.805e-18              lub1 = -7e-25                 wub1 = 0

+pub1 = 6e-31                  uc1 = -1.68e-10               at = -9.9e3

+prt = 0                       xti = 3                       saref = 5e-6

+sbref = 5e-6                  wlod = 0                      ku0 = 0

+kvsat = 0                     kvth0 = 0                     tku0 = 0

+llodku0 = 0                   wlodku0 = 0                   llodvth = 0

+wlodvth = 0                   lku0 = 0                      wku0 = 0

+pku0 = 0                      lkvth0 = 0                    wkvth0 = 0

+pkvth0 = 0                    stk2 = 0                      lodk2 = 1

+steta0 = 0                    lodeta0 = 1                   agidl = 0

+bgidl = 2.3e9                 cgidl = 0.5                   egidl = 0.8

+type = p

//  *************************************************************

//  *                                                           *

//  *               5.0V low vt NMOS DEVICES MODEL                     *

//  *                                                           *

//  *************************************************************

model n5l bsim3v3

+version = 3.2                 binunit = 2                   mobmod = 1

+capmod = 3                    nqsmod = 0                    igcmod = 0

+igbmod = 0                    lmin = 0.5E-6                 lmax = 200.001E-6

+wmin = 0.4E-6                 wmax = 200.001E-6              tox = 1.56e-8+dtoxn5l

+xj = 1e-7                     nch = 1e17                    wint = 0

+wl = -1e-14                   wln = 1                       ww = -2e-14

+wwn = 1                       wwl = 1.15e-20                lint = 0

+ll = 2e-14                    lln = 1                       lw = 0

+lwn = 1                       lwl = -1e-20                  dwg = 0

+dwb = 0                       xl = 6e-9+dxln5l              xw = 7e-9+dxwn5l

+vth0 = 0.62+dvtn5l            lvth0 = 6.5e-8                wvth0 = -2e-8

+pvth0 = -7e-15                k1 = 1.047                    lk1 = -9.4e-8

+wk1 = 0                       pk1 = 0                       k2 = -0.0186

+lk2 = 1.395e-8                wk2 = 0                       pk2 = 2.79e-15

+k3 = 0                        k3b = 0                       w0 = 2.5e-6

+nlx = 1.74e-7                 dvt0 = 2.2                    dvt1 = 0.53

+dvt2 = -0.032                 dvt0w = 0                     dvt1w = 5.3e6

+dvt2w = -0.032                ngate = 1e23                  u0 = 0.04445

+lu0 = 9.695e-9                wu0 = -6.5175e-9              pu0 = 2.4e-15

+ua = -8.09e-10                lua = 8.7525e-16              wua = 1.08945e-15

+pua = -9.8135e-22             ub = 3.241599e-18             lub = -3.979413e-24

+wub = -1.582863e-24           pub = 1.357621e-30            uc = 6.48375e-11

+luc = -2.7528e-16             wuc = -4.48875e-17            puc = 4.58875e-23

+vsat = ( 9.06e4*dvsatn5l) + 0 pvsat = 5.1e-9                a0 = 1.1

+la0 = -2.3e-7                 wa0 = 0                       pa0 = 1.25e-13

+ags = 0.1215                  lags = -1.982125e-7           wags = -8e-9

+pags = -6.225e-14             b0 = 0                        b1 = 0

+keta = 0.011505               lketa = 4.8e-9                wketa = 5e-9

+pketa = 2.1736e-14            a1 = 0                        a2 = 1

+rdsw = 2.25e3                 prdsw = -1.7e-10              prwb = 0

+prwg = 0                      wr = 1                        voff = -0.08

+nfactor = 1                   lnfactor = 8.25e-8            wnfactor = 0

+pnfactor = 0                  eta0 = 0.056                  etab = -0.0385

+dsub = 0.56                   cit = 0                       cdsc = 2.4e-4

+cdscb = 0                     cdscd = 0                     pclm = 1.95

+ppclm = 0                     pdiblc1 = 2.775558e-17        pdiblc2 = 2.15e-3

+pdiblcb = 0                   drout = 0.56                  pscbe1 = 5.512e8

+ppscbe1 = 0                   pscbe2 = 1e-5                 pvag = 0

+delta = 0.038                 pdelta = 1.2e-14              alpha0 = -6.7e-7

+alpha1 = 19.5                 beta0 = 35.5                  nigc = 1

+aigc = 0.012                  bigc = 2.8e-3                 cigc = 2e-3

+aigbacc = 0.012               bigbacc = 2.8e-3              cigbacc = 2e-3

+nigbacc = 1                   aigbinv = 0.014               bigbinv = 4e-3

+cigbinv = 4e-3                eigbinv = 1.1                 nigbinv = 3

+ntox = 1                      toxref = 1.5e-8               pigcd = 1

+poxedge = 1                   dlcig = 0                     aigsd = 0.012

+bigsd = 2.8e-3                cigsd = 2e-3                  elm = 0

+xpart = 0                     cgso = cgon5l                 cgdo = cgon5l

+cgbo = 1e-13                  ckappa = 0.6                  cf = 0

+clc = 1e-7                    cle = 0.6                     dlc = 3.5e-8

+dwc = 0                       vfbcv = -1                    cgsl = 7e-11

+cgdl = 7e-11                  acde = 0.5                    calcacm = 1

+ldif = 1.2e-7                 hdif = 3.8e-7                 rsh = 85

+rd = 0                        rs = 0                        rsc = 0

+rdc = 0                       cj = cjn5l                    mj = 0.321

+mjsw = 0.265                  cjsw = cjswn5l                cjswg = cjswgn5l

+mjswg = 1.36                  js = 1e-4                     jsw = 0

+pbsw = 0.569                  pb = 0.681                    pbswg = 1.56

+tnom = 25                     ute = -1.1                    lute = 7.5e-8

+wute = -3e-8                  pute = 0                      kt1 = -0.418

+lkt1 = 0                      wkt1 = -2e-8                  pkt1 = 0

+kt1l = 0                      kt2 = -0.066                  lkt2 = 0

+wkt2 = 0                      pkt2 = 0                      ua1 = 5.11e-9

+lua1 = 0                      wua1 = -6.465e-16             pua1 = 0

+ub1 = -7.61e-18               lub1 = 1.520075e-24           wub1 = 4e-25

+pub1 = -4.45e-31              uc1 = -1.568e-10              luc1 = 0

+wuc1 = 0                      puc1 = 0                      at = 2.805e4

+prt = 880                     xti = 3                       saref = 5e-6

+sbref = 5e-6                  wlod = 0                      ku0 = 0

+kvsat = 0                     kvth0 = 0                     tku0 = 0

+llodku0 = 0                   wlodku0 = 0                   llodvth = 0

+wlodvth = 0                   lku0 = 0                      wku0 = 0

+pku0 = 0                      lkvth0 = 0                    wkvth0 = 0

+pkvth0 = 0                    stk2 = 0                      lodk2 = 1

+steta0 = 0                    lodeta0 = 1                   agidl = 0

+bgidl = 2.3e9                 cgidl = 0.5                   egidl = 0.8

+type = n

//

endsection mos_hv

//  ***********************************************************************

//

//  ******************************************************************************

//  *                               BIPOLAR MODELS                               *

//  ******************************************************************************

//

section bip

//  ******************************************************************************

//  *                    P+/NW/Psub VERTICAL PNP 8X8 BIPOLAR                     *

//  *                        EMITTER AREA :  8X8   UM^2                          *

//  ******************************************************************************

model qp8 bjt type=pnp rbmod=spice

+ struct=vertical

+ tlev=2                    tlevc=1                   iss=0

+ nkf=0.4541                is=1.175e-17 * isa

+ bf=3 * bfa                nf=1.002 * nfa            vaf=174.4

+ ikf=0.0009929             ise=2.379e-17             ne=1.493

+ br=0.1113                 nr=1.003                  var=13.69

+ ikr=0.4446                isc=1.332e-14             nc=1.057

+ rb=103.6 * rba            irb=0.0002008             rbm=1.354*rbma

+ re=9.958 * rea            rc=5.679 * rca            eg=1.17

+ cje=8.062e-14 * cjea      vje=0.859                 mje=0.408

+ cjc=6.798e-14 * cjca      vjc=0.518                 mjc=0.357

+ fc=0.6853                 tbf1=0.006406             xtb=0.0003545

+ tne1=0.0004102            tnf1=7.996e-06            trb1=0.00199

+ tre1=0.007426             trc1=0.002501             tirb1=-0.007

+ trm1=0.014                tvaf1=0.0001892           tikf1=-0.003919

+ tbr1=0.001042             tikr1=0.009993             tvar1=-0.0006897

+ tnr1=0.0001909            tnc1=0.000844

+ ctc=0.00403299            tvjc=0.00380816           tmjc1=-0.000842251

+ cte=0.000968              tvje=0.00163              tmje1=1.752e-5

//

//  ******************************************************************************

//  *                    P+/NW/Psub VERTICAL PNP 8X8 BIPOLAR                     *

//  *                        EMITTER AREA :  8X8   UM^2                          *

//  *                            for pure 3.3V PNP BJT                           *

//  ******************************************************************************

model qpl8 bjt type = pnp

//*** flag parameter ***

+tlev = 0                      tlevc = 1                     tnom = 25

+subs = 1

//*** dc model parameter ***

+is = 7.75e-18*isa             iss = 0                       bf = 2.19*bfa

+nf = 1.004*nfa                vaf = 174                     ikf = 9.9e-4

+ise = 2.379e-17               ne = 1.493                    br = 2.79e-3

+nr = 0.995                    var = 16.3                    ikr = 0.1

+isc = 0                       nc = 2

//*** series resistance related parameter ***

+rb = 103*rba                irb = 2e-4                    rbm = 1.354*rbma

+re = 9.9*rea                rc = 1.5*rca

//*** capacitance parameter ***

+cje = 8.062E-14*cjea        vje = 0.859                   mje = 0.408

+cjc = 6.798E-14*cjca        vjc = 0.518                   mjc = 0.357

+tf = 1e-10                  xtf = 1                       vtf = 10

+itf = 0.1                   xcjc = 1                      fc = 0.6853

+tr = 0                      ptf = 0

//*** temperature coefficient ***

+xtb = 1e-4                    xti = 3                       eg = 1.17

+tis1 = 0                      tis2 = 0                      tbf1 = 6e-3

+tbf2 = 9.3e-6                 tnf1 = -1.075e-4              tvaf1 = -1.5e-3

+tvaf2 = 0                     tikf1 = 0                     tikf2 = 0

+tbr1 = -1.2e-3                tbr2 = 1.079e-5               tnr1 = -1.265308e-4

+tvar1 = -2.1e-3               cte = 0.000968                ctc = 0.00403299

+tvje = 0.00163                tvjc = 0.00380816             tre1 = 0

+tre2 = 0                      trb1 = 0                      trb2 = 0

+trc1 = 0                      trc2 = 0

+tmjc1 = -0.000842251          tmje1 = 1.752e-5

//*** noise parameter ***

+kf = 0                        af = 1

//

endsection bip

//  ******************************************************************************

//  *                                 DIODE MODELS                               *

//  ******************************************************************************

//

section dio

//  ******************************************************************************

//  *                                 N+/PW Diode                                *

//  ******************************************************************************

model dn diode  allow_scaling=1

+ level=3

+ is=8e-07                  rs=9.95e-08               n=1.085

+ tnom=25                   bv=9.3                   ibv=5e-05

+ cj=8.278e-04              cjsw=2.071e-10            mj=0.27

+ mjsw=0.0819               fc=0.5                    eg=1.17

+ vjsw=0.112                ptp=0.0007957             vj=0.633

+ isw=8e-13

+ cta=0.0009707             ctp=0.001322              tlev=0

+ xti=3                     trs=0.00327               tlevc=1

+ pta=0.001717              tbv1=0.004                ns=1.085

+ nz=1.085

+ imax=1e40		    minr=1e-10

//

//  ******************************************************************************

//  *                                                                            *

//  *                                 P+/NW Diode                                *

//  *                                                                            *

//  ******************************************************************************

model dp diode  allow_scaling=1

+ level=3

+ is=4e-07                  rs=8.5e-08                n=1.085

+ tnom=25                   bv=7.25                  ibv=1e-04

+ cj=1.447e-03              cjsw=3.227e-11            mj=0.434

+ mjsw=0.244                fc=0.5                    eg=1.17

+ vjsw=0.714                ptp=1.585e-3              vj=0.908

+ isw=3e-13

+ cta=9.148e-4              ctp=6.631e-4              tlev=0

+ xti=3                     trs=0.00327               tlevc=1

+ pta=1.591e-3              tbv1=0.003                ns=1.085

+ nz=1.085

+ imax=1e40		    minr=1e-10

//

//

//  ******************************************************************************

//  *                                                                            *

//  *                        P+/NW Diode  (5um*5um)                              *

//  *                                                                            *

//  ******************************************************************************

model dp2 diode

+ level=3

+ is=1.6E-7                  rs=5.5E-10                n=0.99

+ tnom=25                   bv=7.3                  ibv=1e-04

+ cj=1.447e-03              cjsw=3.227e-11            mj=0.434

+ mjsw=0.244                fc=0.5                    eg=1.17

+ vjsw=0.714                ptp=1.585e-3              vj=0.908

+ isw=1.6E-14

+ cta=9.148e-4              ctp=6.631e-4              tlev=0

+ xti=3                     trs=-5.3E-4               tlevc=1

+ pta=1.591e-3              tbv1=2.3E-4                ns=1.085

+ nz=1.085

+ imax=1e40		    minr=1e-10

+ ik = 1E8

//

//

// ******************************************************************************

//  *                                                                            *

//  *                             P+/NW Zener Diode                              *

//  *                                                                            *

//  ******************************************************************************

model dz diode  allow_scaling=1

+ level=3

+ is=1.133e-07              rs=1.969e-10              n=0.9838

+ tnom=25                   bv=5.135                 ibv=0.03

+ cj=0.001                  cjsw=3.00e-11             mj=0.43

+ mjsw=0.24                 fc=0.5                    eg=1.17

+ vjsw=0.737                ptp=0.0018                vj=0.90

+ isw=2.311e-13

+ cta=0.00111               ctp=0.000896              tlev=0

+ xti=3                     trs=0.001                 tlevc=1

+ pta=0.0016                tbv1=0.002679             ns=0.9838

+ nz=0.9838

+ imax=1e40		    minr=1e-10

//

endsection dio

//  ***********************************************************************

//  ******************************************************************************

//  *                               Resistor Model                               *

//  ******************************************************************************

section res

subckt rnp ( n1 n2 )

parameters l=1u w=1u rsh=88 * drsh dw=1.36861e-08 ptc1=1.5069e-03 ptc2=3.9586e-07

+ pvc1=1.0638e-03 pvc2=-2.3145e-05 pt=temp tfac=1.0002 + ptc1 * (pt - 25.0) +

+ ptc2 * (pt - 25.0) * (pt - 25.0)

r1 ( n1 n2 ) bsource r=rsh * l / (w - 2 * dw) * (1 + pvc1 * (abs(v(n2,n1)) - 0.5)

+ + pvc2 * (abs(v(n2,n1)) - 0.5) * (abs(v(n2,n1)) - 0.5)) * tfac

ends rnp

subckt rpp ( n1 n2 )

parameters l=1u w=1u rsh=186.7 * drsh dw=-6.9147e-09 ptc1=1.0860e-03 ptc2=1.0409e-06

+ pvc1=2.0704e-03 pvc2=-9.3321e-05 pt=temp tfac=1.0036 + ptc1 * (pt - 25.0) +

+ ptc2 * (pt - 25.0) * (pt - 25.0)

r1 ( n1 n2 ) bsource r=rsh * l / (w - 2 * dw) * (1 + pvc1 * (abs(v(n2,n1)) - 0.5)

+ + pvc2 * (abs(v(n2,n1)) - 0.5) * (abs(v(n2,n1)) - 0.5)) * tfac

ends rpp

subckt rnpl ( n1 n2 )

parameters l=1u w=1u rsh=92.37 * drsh dw=0 ptc1=1.6247e-03 ptc2=3.7831e-07

+ pvc1=1.2574e-03 pvc2=-1.0440e-05 pt=temp tfac=9.9993e-01 + ptc1 * (pt - 25.0)

+ + ptc2 * (pt - 25.0) * (pt - 25.0)

r1 ( n1 n2 ) bsource r=rsh * l / (w - 2 * dw) * (1.00 + pvc1 * (abs(v(n2,n1)) -

+ 0.5) + pvc2 * (abs(v(n2,n1)) - 0.5) * (abs(v(n2,n1)) - 0.5)) * tfac

ends rnpl

subckt rppl ( n1 n2 )

parameters l=1u w=1u rsh=243.62 * drsh dw=0 ptc1=1.168e-03 ptc2=9.7312e-07

+ pvc1=1.8693e-03 pvc2=-1.0071e-04 pt=temp tfac=9.9993e-01 + ptc1 * (pt - 25.0)

+ + ptc2 * (pt - 25.0) * (pt - 25.0)

r1 ( n1 n2 ) bsource r=rsh * l / (w - 2 * dw) * (1.00 + pvc1 * (abs(v(n2,n1)) -

+ 0.5) + pvc2 * (abs(v(n2,n1)) - 0.5) * (abs(v(n2,n1)) - 0.5)) * tfac

ends rppl

subckt rp1 ( n1 n2 )

parameters l=1u w=1u rsh=9.05 * drsh dw=-1.7e-08 ptc1=4.6803e-04 ptc2=5.0612e-07

+ pvc1=4e-03 pvc2=4.5772e-04 pt=temp tfac=1.0036 + ptc1 * (pt - 25.0) + ptc2 *

+ (pt - 25.0) * (pt - 25.0)

r1 ( n1 n2 ) bsource r=rsh * l / (w - 2 * dw) * (1 + pvc1 * (abs(v(n2,n1)) - 0.5)

+ + pvc2 * (abs(v(n2,n1)) - 0.5) * (abs(v(n2,n1)) - 0.5)) * tfac

ends rp1

subckt rnw ( n1 n2 )

parameters l=1u w=1u rsh=1.37e+03 * drsh dw=2.088489e-07 ptc1=3.5649e-03

+ ptc2=1.0389e-05 pvc1=-7e-03 pvc2=-2.0923e-04 vcb1=5.062e-02 vcb2=1.024e-04 pt=temp

+ vb=0 tfac=1.0011 + ptc1 * (pt - 25.0) + ptc2 * (pt - 25.0) * (pt - 25.0)

r1 ( n1 n2 ) bsource r=rsh * l / (w - 2 * dw) * (1.0 + pvc1 * (abs(v(n2,n1)) - 0.5)

+ + pvc2 * (abs(v(n2,n1)) - 0.5) * (abs(v(n2,n1)) - 0.5)) * (1.00 + vcb1 * ((abs(V(n2,n1)) - 0.5) * 0.3 - vb) + vcb2 * ((abs(V(n2,n1)) - 0.5) * 0.3 - vb) * ((abs(V(n2,n1)) - 0.5) * 0.3 - vb) ) * tfac

ends rnw

subckt rhnw ( n1 n2 )

parameters l=1u w=1u rsh=1.6114e+03 * drsh dw=2.31509e-07 ptc1=3.8288e-03

+ ptc2=1.1562e-05 pvc1=-3.249e-03 pvc2=6.690e-04 vcb1=5.876e-02 vcb2=7.603e-04 pt=temp

+ vb=0 tfac=0.99964 + ptc1 * (pt - 25.0) + ptc2 * (pt - 25.0) * (pt - 25.0)

r1 ( n1 n2 ) bsource r=rsh * l / (w - 2 * dw) * (1.0 + pvc1 * (abs(v(n2,n1)) - 0.5)

+ + pvc2 * (abs(v(n2,n1)) - 0.5) * (abs(v(n2,n1)) - 0.5)) * (1.00 + vcb1 * ((abs(V(n2,n1)) - 0.5) * 0.5 - vb) + vcb2 * ((abs(V(n2,n1)) - 0.5) * 0.5 - vb) * ((abs(V(n2,n1)) - 0.5) * 0.5 - vb) ) * tfac

ends rhnw

subckt rhr1k ( n1 n2 )

parameters

+l = length                    w = width                     dw = -2.6366e-8

+pdw1 = -2.9098e-3             pdw2 = -3.5055e-4             ptc1 = -1.4263e-3

+ptc2 = 4.0758e-6              pvc1 = -9.3766e-4             pvc2 = 1.1855e-4

+rsh = 1158*drsh               pt = temp                     tfac = 1.001+ptc1*(pt-25.0)+ptc2*(pt-25.0)*(pt-25.0)

r1  (n1  n2  )  resistor  r=rsh*l/(w-2*dw*(1.001+pdw1*(abs(v(n2,n1)-0.5))+pdw2*(abs(v(n2,n1)-0.5))*(abs(v(n2,n1)-0.5))))*(0.99989+pvc1*(abs(v(n2,n1))-0.5)+pvc2*(abs(v(n2,n1))-0.5)*(abs(v(n2,n1))-0.5))*tfac

ends rhr1k

subckt rhr2k ( n1 n2 )

parameters

+l = length                    w = width                     dw = -3.6411e-8

+pdw1 = -2.7048e-3             pdw2 = -3.0266e-4             ptc1 = -2.3138e-3

+ptc2 = 7.1674e-6              pvc1 = -8.7673e-4             pvc2 = 1.215e-4

+rsh = 2215*drsh               pt = temp                     tfac = 1.002+ptc1*(pt-25.0)+ptc2*(pt-25.0)*(pt-25.0)

r1  (n1  n2  )  resistor  r=rsh*l/(w-2*dw*(1.001+pdw1*(abs(v(n2,n1)-0.5))+pdw2*(abs(v(n2,n1)-0.5))*(abs(v(n2,n1)-0.5))))*(0.99984+pvc1*(abs(v(n2,n1))-0.5)+pvc2*(abs(v(n2,n1))-0.5)*(abs(v(n2,n1))-0.5))*tfac

ends rhr2k

subckt rhr4k ( n1 n2 )

parameters

+l = length                    w = width                     dw = -4.7639e-8

+pdw1 = -5.3911e-3             pdw2 = -4.8933e-5             ptc1 = -3.0758e-3

+ptc2 = 9.9626e-6              pvc1 = -8.8079e-4             pvc2 = 1.265e-4

+rsh = 4036*drsh               pt = temp                     tfac = 1.002+ptc1*(pt-25.0)+ptc2*(pt-25.0)*(pt-25.0)

r1  (n1  n2  )  resistor  r=rsh*l/(w-2*dw*(1.002+pdw1*(abs(v(n2,n1)-0.5))+pdw2*(abs(v(n2,n1)-0.5))*(abs(v(n2,n1)-0.5))))*(0.99975+pvc1*(abs(v(n2,n1))-0.5)+pvc2*(abs(v(n2,n1))-0.5)*(abs(v(n2,n1))-0.5))*tfac

ends rhr4k

subckt rhr10k ( n1 n2 )

parameters

+l = length                    w = width                     dw = -8.7635e-8

+pdw1 = -3.4051e-3             pdw2 = -7.6159e-5             ptc1 = -4.6399e-3

+ptc2 = 1.7868e-5              pvc1 = -6.2306e-4             pvc2 = 1.3242e-4

+rsh = 10100*drsh              pt = temp                     tfac = 1.008+ptc1*(pt-25.0)+ptc2*(pt-25.0)*(pt-25.0)

r1  (n1  n2  )  resistor  r=rsh*l/(w-2*dw*(1.002+pdw1*(abs(v(n2,n1)-0.5))+pdw2*(abs(v(n2,n1)-0.5))*(abs(v(n2,n1)-0.5))))*(0.99969+pvc1*(abs(v(n2,n1))-0.5)+pvc2*(abs(v(n2,n1))-0.5)*(abs(v(n2,n1))-0.5))*tfac

ends rhr10k

subckt rp2 ( n1 n2 )

parameters l=1u w=1u rsh=40.82 * drsh ptc1=6.3014e-04 ptc2=4.3384e-07 pvc1=3.4283e-04

+ pvc2=4.4131e-04 pt=temp tfac=1.0 + ptc1 * (pt - 25.0) + ptc2 * (pt - 25.0) *

+ (pt - 25.0)

r1 ( n1 n2 ) bsource r=rsh * l / (w) * (1.0 + pvc1 * (abs(v(n2,n1)) - 0.5) + pvc2

+ * (abs(v(n2,n1)) - 0.5) * (abs(v(n2,n1)) - 0.5)) * tfac

ends rp2

subckt rlp2 ( n1 n2 )

parameters

+l = length                    w = width                     dw = -9.95683e-9

+ptc1 = -4.5303e-4             ptc2 = 9.0114e-7              pvc1 = -9e-4

+pvc2 = 1e-4                   rsh = 324*drsh                pt = temp

+tfac = 0.98782+ptc1*(pt-25.0)+ptc2*(pt-25.0)*(pt-25.0)

r1  (n1  n2  )  bsource  r = rsh*l/(w-2*dw)*(0.9995+pvc1*(abs(v(n2,n1))-0.5)+pvc2*(abs(v(n2,n1))-0.5)*(abs(v(n2,n1))-0.5))*tfac

ends rlp2

subckt rm1 ( n1 n2 )

parameters l=1u w=1u rsh=0.06 * drsh dw=0 ptc1=3.84e-3 ptc2=-1.06e-6 pvc1=0

+ pvc2=0 pt=temp tfac=1.0 + ptc1 * (pt - 25.0) + ptc2 * (pt - 25.0) * (pt - 25.0)

r1 ( n1 n2 ) bsource r=rsh * l / (w - 2 * dw) * (1 + pvc1 * (abs(v(n2,n1)) - 0.5)

+ + pvc2 * (abs(v(n2,n1)) - 0.5) * (abs(v(n2,n1)) - 0.5)) * tfac

ends rm1

subckt rm2 ( n1 n2 )

parameters l=1u w=1u rsh=0.05 * drsh dw=0 ptc1=3.82e-3 ptc2=-1.53e-6 pvc1=0

+ pvc2=0 pt=temp tfac=1.0 + ptc1 * (pt - 25.0) + ptc2 * (pt - 25.0) * (pt - 25.0)

r1 ( n1 n2 ) bsource r=rsh * l / (w - 2 * dw) * (1 + pvc1 * (abs(v(n2,n1)) - 0.5)

+ + pvc2 * (abs(v(n2,n1)) - 0.5) * (abs(v(n2,n1)) - 0.5)) * tfac

ends rm2

subckt rtm6k ( n1 n2 )

parameters

+l = length                    w = width                     dw = 0

+pdw1 = 0                      pdw2 = 0                      ptc1 = 3.77e-3

+ptc2 = -2.51e-7               pvc1 = 0                      pvc2 = 0

+rsh = 0.05*drsh               pt = temp                     tfac = 1.0+ptc1*(pt-25.0)+ptc2*(pt-25.0)*(pt-25.0)

r1  (n1  n2  )  resistor  r = rsh*l/(w-2*dw)*(1+pvc1*(abs(v(n2,n1)))+pvc2*(abs(v(n2,n1)))*(abs(v(n2,n1))))*tfac

ends rtm6k

subckt rtm8k ( n1 n2 )

parameters

+l = length                    w = width                     dw = 0

+pdw1 = 0                      pdw2 = 0                      ptc1 = 3.77e-3

+ptc2 = -2.51e-7               pvc1 = 0                      pvc2 = 0

+rsh = 3.01e-02*drsh           pt = temp                     tfac = 1.0+ptc1*(pt-25.0)+ptc2*(pt-25.0)*(pt-25.0)

r1  (n1  n2  )  resistor  r = rsh*l/(w-2*dw)*(1+pvc1*(abs(v(n2,n1)))+pvc2*(abs(v(n2,n1)))*(abs(v(n2,n1))))*tfac

ends rtm8k

subckt rtm20k ( n1 n2 )

parameters

+l = length                    w = width                     dw = 0

+ptc1 = 1.83e-3                ptc2 = 2.11e-6                pvc1 = 0

+pvc2 = 0                      rsh = 0.014*drsh              pt = temp

+tfac = 1.0+ptc1*(pt-25.0)+ptc2*(pt-25.0)*(pt-25.0)

r1  (n1  n2  )  resistor  r = rsh*l/(w-2*dw)*(1+pvc1*(abs(v(n2,n1)))+pvc2*(abs(v(n2,n1)))*(abs(v(n2,n1))))*tfac

ends rtm20k

subckt rtm30k ( n1 n2 )

parameters

+l = length                    w = width                     dw = 0

+ptc1 = 3.2e-3                 ptc2 = -6e-6                  pvc1 = 0

+pvc2 = 0                      rsh = 0.00941*drsh            pt = temp

+tfac = 1.0+ptc1*(pt-25.0)+ptc2*(pt-25.0)*(pt-25.0)

r1  (n1  n2  )  resistor  r = rsh*l/(w-2*dw)*(1+pvc1*(abs(v(n2,n1)))+pvc2*(abs(v(n2,n1)))*(abs(v(n2,n1))))*tfac

ends rtm30k

endsection res

//  ******************************************************************************

//  *                                                                            *

//  *                               POP capacitance Model                        *

//  *                                                                            *

//  ******************************************************************************

section cap

subckt cp ( n1 n2 )

parameters l=1u w=1u ucap=7.83188e-04 * ducap ptc1=2.4e-05 ptc2=3.13e-08

+ pvc1=-2.9568e-04 pvc2=-1.1863e-05 pt=temp tfac=1.0 + ptc1 * (pt - 25.0) + ptc2

+ * (pt - 25.0) * (pt - 25.0)

c1 ( n1 n2 ) bsource c=ucap * l * w * (1 + pvc1 * (v(n1,n2) - 0.1) + pvc2 * (v(n1,n2)

+ - 0.1) * (v(n1,n2) - 0.1)) * tfac

ends cp

endsection cap

//  *****************************************************************************************************************

endlibrary MyLib